# Python Radio 36: Mesh Networking

Simon Quellen Field

Follow

16 min read

·

Apr 15, 2025

Listen

Share

More

Get the word out to nodes you can’t see.

Press enter or click to view image in full size

MidJourney

Here in California’s San Francisco Bay Area, we have a very active group using the Meshtastic software to form a mesh network of LoRa nodes.

I can send messages to San Francisco 54 miles away using only a few milliwatts of power and a tiny solar-powered computer that cost me about $30 to put together.

The Meshtastic software is very robust and capable, with lots of bells and whistles. I thought it would be good fun to build a much simpler, smaller system using nodes that cost $5 (although I have found two sites on Aliexpress.com that offer them for 99 cents, but only one to a customer, presumably as a marketing program).

By using the built-in WiFi instead of a separate LoRa radio chip, we save cost at the expense of long distance but gain a lot of extra bandwidth.

The board I chose is one of my new favorites, the ESP32S3 Supermini. It has 4 megabytes of flash memory, 2.5 megabytes of RAM, runs at 240 megahertz, has two 32-bit processors (and a third 32-bit low-power processor), WiFi, Bluetooth, 24 GPIO pins, an RGB LED, and a Type-C connector for power and programming.

And the little thing is the size of my thumbnail:

Press enter or click to view image in full size

Photo by author

That tiny little surface-mounted red device labeled C3 is the antenna. For most uses, you would probably want to solder a 6-inch wire to that to get better range, but it reaches all over my house through walls, so it is fine for most things.

We use the tiny antenna to make testing our mesh network easier. We want to have some nodes out of reach of at least one node to make sure we are actually networking and not talking to them directly.

### The Design

The goal of the project is to make it simple.

So we will not do any fancy routing. We will broadcast each message to all the neighbors we can reach. They will receive it and broadcast it to everyone they can reach.

This could result in a catastrophic explosion of packets, saturating the network quickly, except for two things:

- Each packet has a time-to-live number that is decremented before retransmitting the packet. If the number gets to zero, the node drops the packet.

- Each packet has a sequence number, and if a node sees a sequence number it has already processed, it drops the packet. We keep the last 20 sequence numbers in a circular list.

That’s the whole design.

Much of the code is just a simple web server.

### The Code

We have two support modules, connect.py and uping.py. We don’t actually need uping.py, but it made debugging the code easier at one point. It ‘ping’s an IP address to verify the connection.

Here is connect.py, the module that set up the WiFi access point and connects to the other nodes’ access points:

class Connect:
  def __init__( self ):
    from network import WLAN, STA_IF, AP_IF, AUTH_OPEN
    from random import randint

    self.ssid = ""
    self.verbose = False
    self.network_list = []

    self.sta = WLAN( STA_IF )
    self.sta.active( False )
    self.sta.active( True )

    self.ap = WLAN( AP_IF )
    self.ap.active( False )
    self.ap.active( True )
    self.mac = self.ap.config('mac')

    self.mac_print = ""
    for m in self.mac:
        self.mac_print += hex(m)[2:] + ":"
    self.mac_print = self.mac_print[:-1]

    self.mac_str = ""
    for m in self.mac[3:]:
        self.mac_str += hex(m)[2:]

    ip1 = 10
    ip2 = self.mac[-2]
    ip3 = self.mac[-1]
    ip4 = 1

    ip_str = str(ip1) + "." + str(ip2) + "." + str(ip3) + "." + str(ip4)
    ip_mask = "255.255.255.0"

    self.ap.ifconfig((ip_str, ip_mask, ip_str, "8.8.8.8"))
    self.ap_ip = self.ap.ifconfig()[2]
    self.sta_host_ip = False
    self.sta_client_ip = False
    self.who_am_i = "mesh_" + self.mac_str

    self.ap.config( essid=self.who_am_i, authmode=AUTH_OPEN )
    self.verbose and print(f"Serving as {self.who_am_i} on subnet {self.ap_ip}")

  def connect( self ):
    from network import WLAN, STA_IF, AUTH_OPEN
    from uping import ping
    from time import sleep

    self.verbose and print( "Connect" )

    for count in range(5):
      self.sta.active( True )
      try:
        self.network_list = self.sta.scan()
        # Sort by RSSI
        self.network_list = sorted(self.network_list, key=lambda x: x[3], reverse=True)
        break
      except Exception as e:
        print( "scan failed:", e )
        self.sta.disconnect()
        self.sta.active( False )
        self.sta = WLAN( STA_IF )
        self.sta.active( True )

    try:
      if not self.sta.isconnected():
        for net in self.network_list:
          self.ssid = net[0].decode("utf-8")
          if "mesh_" in self.ssid:
            try:
              self.sta.connect( self.ssid, "" )
              count = 0
              while not self.sta.isconnected() and count < 10:
                sleep( 1 )
                count += 1
              if not self.sta.isconnected():
                continue
              ip = self.sta.ifconfig()[0]
              self.sta.config(dhcp_hostname=self.who_am_i)
              self.sta_host_ip = self.sta.ifconfig()[2]
              self.sta_client_ip = self.sta.ifconfig()[0]
              p = ping(str(self.sta_host_ip), quiet=True)
              if p[1] == 0:
                self.verbose and print(f"Can't ping {self.sta_host_ip}")
              else:
                # self.verbose and print(f"Pinged: {p}")
                # self.verbose and print(f"Ifconfig: {self.sta.ifconfig()}")
                # self.verbose and print(f"Connected to host at subnet {self.sta_host_ip}")
                pass
              return True
            except Exception as e:
              print( "Error in connect():", e )
              pass
      else:
        if self.verbose:
          print( "Already connected" )
        return True
    except Exception as e:
      print( "connect():", e )
    self.ssid = ""
    self.verbose and print(f"No mesh nodes found")
    return False

  def reconnect( self ):
    # self.verbose and print( "Reconnecting" )
    if self.sta.isconnected():
        self.sta.disconnect()
        self.sta.active( False )
    return self.connect()

It sets up an access point, advertising itself as mesh_XXXXXX, where the Xs are the last 3 bytes of the MAC address, in hexadecimal.

It then searches for other SSIDs that start with “mesh_” and tries to connect to them.

The first node to power up will not find any. But the second one to power up within reach of the signal will find this node and connect.

The uping.py module implements the ping command:

# µPing (MicroPing) for MicroPython
# copyright (c) 2018 Shawwwn <shawwwn1@gmail.com>
# License: MIT

# Internet Checksum Algorithm
# Author: Olav Morken
# https://github.com/olavmrk/python-ping/blob/master/ping.py
# @data: bytes
def checksum(data):
    if len(data) & 0x1: # Odd number of bytes
        data += b'\0'
    cs = 0
    for pos in range(0, len(data), 2):
        b1 = data[pos]
        b2 = data[pos + 1]
        cs += (b1 << 8) + b2
    while cs >= 0x10000:
        cs = (cs & 0xffff) + (cs >> 16)
    cs = ~cs & 0xffff
    return cs

def ping(host, count=4, timeout=5000, interval=10, quiet=False, size=64):
    import utime
    import uselect
    import uctypes
    import usocket
    import ustruct
    import urandom

    # prepare packet
    assert size >= 16, "pkt size too small"
    pkt = b'Q'*size
    pkt_desc = {
        "type": uctypes.UINT8 | 0,
        "code": uctypes.UINT8 | 1,
        "checksum": uctypes.UINT16 | 2,
        "id": uctypes.UINT16 | 4,
        "seq": uctypes.INT16 | 6,
        "timestamp": uctypes.UINT64 | 8,
    } # packet header descriptor
    h = uctypes.struct(uctypes.addressof(pkt), pkt_desc, uctypes.BIG_ENDIAN)
    h.type = 8 # ICMP_ECHO_REQUEST
    h.code = 0
    h.checksum = 0
    h.id = urandom.getrandbits(16)
    h.seq = 1

    # init socket
    sock = usocket.socket(usocket.AF_INET, usocket.SOCK_RAW, 1)
    sock.setblocking(0)
    sock.settimeout(timeout/1000)
    addr = usocket.getaddrinfo(host, 1)[0][-1][0] # ip address
    sock.connect((addr, 1))
    not quiet and print("PING %s (%s): %u data bytes" % (host, addr, len(pkt)))

    seqs = list(range(1, count+1)) # [1,2,...,count]
    c = 1
    t = 0
    n_trans = 0
    n_recv = 0
    finish = False
    while t < timeout:
        if t==interval and c<=count:
            # send packet
            h.checksum = 0
            h.seq = c
            h.timestamp = utime.ticks_us()
            h.checksum = checksum(pkt)
            if sock.send(pkt) == size:
                n_trans += 1
                t = 0 # reset timeout
            else:
                seqs.remove(c)
            c += 1

        # recv packet
        while 1:
            socks, _, _ = uselect.select([sock], [], [], 0)
            if socks:
                resp = socks[0].recv(4096)
                resp_mv = memoryview(resp)
                h2 = uctypes.struct(uctypes.addressof(resp_mv[20:]), pkt_desc, uctypes.BIG_ENDIAN)
                # TODO: validate checksum (optional)
                seq = h2.seq
                if h2.type==0 and h2.id==h.id and (seq in seqs): # 0: ICMP_ECHO_REPLY
                    t_elasped = (utime.ticks_us()-h2.timestamp) / 1000
                    ttl = ustruct.unpack('!B', resp_mv[8:9])[0] # time-to-live
                    n_recv += 1
                    not quiet and print("%u bytes from %s: icmp_seq=%u, ttl=%u, time=%f ms" % (len(resp), addr, seq, ttl, t_elasped))
                    seqs.remove(seq)
                    if len(seqs) == 0:
                        finish = True
                        break
            else:
                break

        if finish:
            break

        utime.sleep_ms(1)
        t += 1

    # close
    sock.close()
    ret = (n_trans, n_recv)
    not quiet and print("%u packets transmitted, %u packets received" % (n_trans, n_recv))
    return (n_trans, n_recv)

Unlike the other modules, I didn’t write this one. But it came in quite handy, so I share it here.

### The Main Program

As you might expect, most of the work is done in the main program. Some of it is just a simple web server, something we’ve covered earlier.

The unquote(), handle_query(), decode_path(), req_handler(), and client_handler() process packets that come in and parse the few HTTP protocols we accept. There is nothing “mesh networky” about them.

The dup_sequence() method handles the circular list of sequence numbers we’ve seen. When it finds a duplicate, it returns True to drop the packet.

The show_neighbors() method is for debugging. It prints the neighbor list on the console.

The whodat dictionary helps keep track of the names of the nodes. It is also just for debugging, and it occasionally gets it wrong in non-critical ways.

When a node connects to our node, add_neighbor() puts it into a list of neighbors. It ignores neighbors we already have in the list. Importantly, it calls greet() to send a message back to the connecting node, telling it who we are, and collecting similar information in the reply.

Before I go any further, here is the code:

from uasyncio import get_event_loop
from connect import Connect
from time import sleep
from machine import Pin
from neopixel import NeoPixel
from socket import getaddrinfo, socket, AF_INET, SOCK_STREAM, SOL_SOCKET, SO_REUSEADDR, SOL_SOCKET
from whoami import WhoAmI
from sys import print_exception
from random import randint

w = WhoAmI()
rgb = NeoPixel(Pin(w.neo_pin(), Pin.OUT), 1)

def unquote(string):
    if not string:
        return b""

    if isinstance(string, str):
        string = string.encode("utf-8")

    bits = string.split(b"%")
    if len(bits) == 1:
        return string

    res = bytearray(bits[0])
    append = res.append
    extend = res.extend

    for item in bits[1:]:
        try:
            append(int(item[:2], 16))
            extend(item[2:])
        except KeyError:
            append(b"%")
            extend(item)

    return bytes(res).decode("utf-8")

class Mesh:
    def __init__(self):
        self.verbose = False
        self.neighbors = []
        self.query = ""
        self.sequence_number = randint(1, 100_000_000)
        self.sequences = [0] * 20
        self.seq_count = 0
        self.path = ""
        self.port = 80
        self.whodat = {}
        self.con = None
        self.name = w.name()
        self.addr = getaddrinfo("0.0.0.0", self.port)[0][-1]
        self.srv = socket(AF_INET, SOCK_STREAM)
        self.srv.setsockopt(SOL_SOCKET, SO_REUSEADDR, 1)
        self.srv.bind(self.addr)
        self.srv.listen(10) # at most 10 clients
        self.srv.setblocking(False)
        self.srv.setsockopt(SOL_SOCKET, 20, self.client_handler)

    def dup_sequence(self, seq):
        for s in self.sequences:
            if seq == s:
                return True
        self.sequences[self.seq_count] = seq
        self.seq_count += 1
        self.seq_count %= 20
        return False

    def show_neighbors(self):
        print("Neighbors:")
        print("----------")

        for n in self.neighbors:
            ssid, ip, name = n
            if ip == self.con.sta_host_ip:
                print(f"Neighbor '{ssid}' '{ip}' '{name}' [Our host]")
            else:
                print(f"Neighbor '{ssid}' '{ip}' '{name}'")

        for n in self.whodat.values():
            his_name, his_ip = n
            print(f"Client '{his_name}' '{his_ip}'")

        print("----------")

    def who_name(self, ap_ip, sta_ip, name):
        if name != "unknown":
            for n in self.whodat.values():
                his_name, his_ip = n
                if ap_ip[:-2] == his_ip[:-2]:
                    self.whodat[his_ip] = (name, his_ip)
                if sta_ip and sta_ip[:-2] == his_ip[:-2]:
                    self.whodat[his_ip] = (name, his_ip)


    def add_neighbor(self, ip, ssid="no ssid", name="unknown"):
        self.who_name(ip, None, name)
        # self.verbose and print(f"Adding neighbor: {ip} {ssid} {name}")
        for i, n in enumerate(self.neighbors):
            n_ssid, n_ip, n_name = n
            # self.verbose and print(f"Checking neighbor: {n_ip} {n_ssid} {n_name}")
            if n_ip == ip:                  # We've seen this before
                # self.verbose and print(f"We already have this neighbor")
                if n_name == "unknown":
                    self.neighbors[i][2] = name
                if n_ssid == "no ssid":
                    self.neighbors[i][0] = ssid
                return

        if ip == self.con.ap_ip:            # We are not our own neighbor
            # self.verbose and print(f"We are not our own neighbor")
            return

        try:
            self.greet(ip, name)
        except Exception as e:
            self.verbose and print(f"Exception in connecting: {e}")
            print_exception(e)
            pass
        for n in self.neighbors:
            n_ssid, n_ip, n_name = n
            if n_ssid == ssid and n_ip = ip and n_name == name:
                return
        self.verbose and print(f"{w.name()}: Added neighbor {ssid} {ip} {name}")
        self.neighbors.append([ssid, ip, name])

    def execute(self, payload):
        if payload.get("rgb", False):
            i = payload["rgb"]
            r = (i >> 16) & 0xFF
            g = (i >>  8) & 0xFF
            b = (i      ) & 0xFF
            rgb[0] = (r, g, b)
            rgb.write()
            name = payload["name"]
            ap_ip, sta_ip = payload["origin"]
            self.who_name(ap_ip, sta_ip, name)
        elif payload.get("text", False):
            print()
            msg = payload["message"]
            who = payload["to"]
            name = payload["name"]
            print(f"Message from {name} to {who}:")
            print(msg)
            print()
            pass
        elif payload.get("greet", False):
            name = payload["name"]
            ap_ip, sta_ip = payload["origin"]
            self.who_name(ap_ip, sta_ip, name)
        else:
            self.verbose and print(f"Execute({payload})")

    def colors(self):
        import json
        from random import randint
        r = randint(0, 255)
        g = randint(0, 255)
        b = randint(0, 255)
        for n in self.neighbors:
            ssid, ip, name = n
            payload = {
                    "name": w.name(),
                    "to": name,
                    "ssid": self.con.who_am_i,
                    "rgb": (r << 16) + (g << 8) + b,
                    "time_to_live": 1,
                    "direct": self.con.ap_ip,
                    "origin": (self.con.ap_ip, self.con.sta_client_ip)
                    }
            self.direct_msg(ip, "none", json.dumps(payload))

    def greet(self, ip, name):
        import json
        # self.verbose and print(f"Greeting {name} at {ip}")
        payload = {
                "name": w.name(),
                "to": name,
                "ssid": self.con.who_am_i,
                "greet": True,
                "time_to_live": 0,
                "direct": self.con.ap_ip,
                "origin": (self.con.ap_ip, self.con.sta_client_ip)}
        self.direct_msg(ip, "none", json.dumps(payload))

    def direct(self, ip):
        if ip == self.con.sta_host_ip:
            # self.verbose and print(f"Sending to our host")
            return True
        for n in self.whodat.values():
            his_name, his_ip = n
            if ip == his_ip:
                # self.verbose and print(f"Sending to our client {his_name} at {his_ip}")
                return True
        subnet_ip = ip[:-2]
        subnet_ap = self.con.ap_ip[:-2]
        if subnet_ip == subnet_ap:
            # self.verbose and print(f"Sending to same subnet as our ap: {subnet_ip}")
            return True
        return False

    def direct_msg(self, ip, file, payload_string):
        from urequests import get
        from random import randint
        import json

        self.sequence_number += randint(1, 1_000)
        payload = json.loads(payload_string)
        payload["direct"] = self.con.ap_ip
        payload["sequence"] = self.sequence_number
        payload_string = json.dumps(payload)

        # self.verbose and print(f"My ip addresses: {self.con.ap_ip} {self.con.sta_client_ip}")
        if not self.direct(ip):
            # self.verbose and print(f"Can't reach {ip} directly")
            return

        for count in range(5):
            try:
                get_str = f"http://{ip}/{file}?packet={payload_string}"
                response = get(get_str, timeout=randint(1, 10))
                if response.status_code != 200:
                    response.close()
                    continue
                if response.text[0] == "{":
                    p = json.loads(response.text)
                    self.add_neighbor(p["origin"][0], p["ssid"], p["name"])
                response.close()
                return
            except OSError as e:
                if e.args[0] == 104: # ECONNRESET
                    # self.verbose and print(f"Connection to {ip} reset")
                    pass
                elif e.args[0] == 113: # ECONNABORTED
                    # self.verbose and print(f"Connection to {ip} aborted")
                    pass
                elif e.args[0] == 116: # ETIMEDOUT
                    # self.verbose and print(f"Connection to {ip} timed out")
                    pass
                else:
                    print(f"Exception in get: {e}")
                    print_exception(e)
            except Exception as e:
                print(f"Exception in get: {e}")
                print_exception(e)

    def broadcast(self, payload_string):
        from urequests import get
        self.show_neighbors()
        print()
        print("***************************************************")
        print(f"Broadcast:")
        for n in self.neighbors:
            ssid, ip, name = n
            # self.verbose and print(f"Send to {name} at {ip} ", end="")
            # self.verbose and print(payload_string)
            self.direct_msg(ip, "none", payload_string)
        # for n in self.whodat.values():
        #     his_name, his_ip = n
        #     self.verbose and print(f"Send to {his_name} at {his_ip} ")
        #     self.direct_msg(his_ip, "none", payload_string)
        print("***************************************************")
        print()

    def handle_query(self):
        import json
        command, payload_string = self.query.split("=")
        payload_string = unquote(payload_string)
        if command == "packet":
            payload = json.loads(payload_string)
            name = payload.get("name", "unknown")
            ssid = payload.get("ssid", "no ssid")
            to = payload.get("to", "unknown")
            seq = payload.get("sequence", False)
            if self.dup_sequence(seq):
                # self.verbose and print("We already saw this message:", payload_string)
                return
            direct = payload.get("direct", "unknown")
            its_from_ap, its_from_sta = payload.get("origin", ("unknown", "unknown"))
            ttl = payload.get("time_to_live", 0)
            # self.verbose and print("Query:", name, ssid, to, its_from_ap, its_from_sta)

            self.add_neighbor(its_from_ap, ssid, name)
            self.add_neighbor(its_from_sta, ssid, name)

            if to == w.name():                       # It's for me
                self.execute(payload)
            elif ttl > 0:
                ttl -= 1
                payload["time_to_live"] = ttl
                payload_string = json.dumps(payload)
                if to == "all":
                    self.broadcast(payload_string)
                else:
                    # Here is where we would put the routing code
                    self.broadcast(payload_string)
        elif command == "text":
            payload = json.loads(payload_string)
            text = payload.get("text", "")
            who = payload.get("to", "")
            its_from = payload.get("from", "")
            message = payload.get("message", "")
            seq = payload.get("sequence", False)
            if self.dup_sequence(seq):
                # self.verbose and print("We already saw this message:", payload_string)
                return
            print()
            print("Message:")
            print(payload_string)
            payload = {
                "text": True,
                "message": message,
                "name": its_from,
                "to": who,
                "time_to_live": 3,
                "direct": self.con.ap_ip,
                "origin": (self.con.ap_ip, self.con.sta_client_ip)}
            self.broadcast(json.dumps(payload))
            print()
        else:
            self.verbose and print("Unexpected query:", self.query)
        return None

    def send_file(self, client_socket):
        from ubinascii import hexlify
        # self.verbose and print("Send file:", self.path)
        if self.path:
            if self.path == "/index.html":
                style = "<style>\r\n      th, td\r\n      {\r\n        padding: 5px;\r\n        spacing: 5px;\r\n      }\r\n    </style>"
                my_info = f"<h2>{w.name()}: ap={self.con.ap_ip} sta={self.con.sta_client_ip} on {self.con.who_am_i}</h2><p/>"
                neighbors = "<table border=1>"
                for n in self.neighbors:
                    ssid, ip, name = n
                    neighbors += f"\r\n      <tr>\r\n        <td>{name}</td>\r\n        <td>{ip}</td>\r\n        <td>{ssid}</td>\r\n      </tr>"
                for n in self.whodat.values():
                    his_name, his_ip = n
                    neighbors += f"\r\n      <tr>\r\n        <td>Child</td>\r\n        <td>{his_ip}</td>\r\n        <td>{his_name}</td>\r\n      </tr>"
                neighbors += "\r\n    </table>"
                station_str = "<p/>Stations:\r\n"
                stations = self.con.ap.status("stations")
                for s in stations:
                    mac = hexlify(s[0]).decode()[4:]
                    station_str += f"    <br/>{mac}\r\n"
                r, g, b = rgb[0]
                rgb_str = f"(r={r}, g={g}, b={b})"
                script = """
    <script>
      sequence_number = Math.round(Math.random() * 10000);
      function sendTextToServer() {
        sequence_number++;
        const who = document.getElementById("who").value;
        const from = document.getElementById("from").value;
        const textToSend = document.getElementById("textInput").value;
        packet = { "message": encodeURIComponent(textToSend), "to": who, "from": from, "sequence": sequence_number }
        const url = "/text?text=" + JSON.stringify(packet);
        fetch(url, { method: 'GET', })
        .then(response => {
          if (!response.ok) { throw new Error("HTTP error! status: " + response.status); }
          return response.text();
          })
          .then(data => {
            document.getElementById("responseFromServer").textContent = "Server Response: " + data;
            document.getElementById("textInput").value = "";
          })
          .catch(error => {
            console.error("Error sending text:", error);
            document.getElementById("responseFromServer").textContent = "Error: " + error.message;
          });
        }
    </script>
"""
                file = f"""
<html>
  <head>
    <title>Mesh</title>
    {style}
    {script}
  </head>
  <body>
    {my_info}
    <h3>Mesh: {w.name()}'s neighbors:</h3>
    {neighbors}
    {station_str}
    <h3>LED:</h3>
    RGB is {rgb_str}
    <p/>
    <form id="textForm">
      <div>
        <label for="who">Send to:</label>
        <input type="text" id="who" name="text" placeholder="all">
        <br/>
        <label for="from">Send from:</label>
        <input type="text" id="from" name="text" placeholder="Joe">
        <br/>
        <label for="textInput">Enter text to send:</label>
        <input type="text" id="textInput" name="text" required>
      </div>
      <button type="button" onclick="sendTextToServer()">Send Text</button>
    </form>
    <div id="responseFromServer"></div>
  </body>
</html>
"""
                client_socket.write(f"HTTP/1.1 200 OK\r\nContent-Length: {len(file)}\r\nContent-Type: text/html\r\n\r\n")
                client_socket.write(file)
            elif self.path == "/text":
                file = "Received text message"
                client_socket.write(f"HTTP/1.1 200 OK\r\nContent-Length: {len(file)}\r\nContent-Type: text/html\r\n\r\n")
                client_socket.write(file)
            elif self.path == "/none":
                import json
                payload = {
                    "name": w.name(),
                    "ssid": self.con.who_am_i,
                    "direct": self.con.ap_ip,
                    "origin": (self.con.ap_ip, self.con.sta_client_ip)}
                file = json.dumps(payload)
                client_socket.write(f"HTTP/1.1 200 OK\r\nContent-Length: {len(file)}\r\nContent-Type: text/html\r\n\r\n")
                client_socket.write(file)
            else:
                with open(self.path, "rb") as f:
                    file = f.read()
                    client_socket.write(f"HTTP/1.1 200 OK\r\nContent-Length: {len(file)}\r\nContent-Type: text/html\r\n\r\n")
                    client_socket.write(file)

    def decode_path(self, req):
        if not req:
            return "/"

        self.path = None
        cmd, headers = req.split("\r\n", 1)
        if cmd:
            s = cmd.split(" HTTP/")
            cmd = s[0]
            self.query = ""
            r = cmd.find("?")
            if r > 0:
                self.query = cmd[r+1:]
                cmd = cmd[:r]

            self.method, self.path = cmd.split(" ")

            if self.path == "/":
                self.path = "/index.html"

        return self.path

    def req_handler(self, client_socket):
        try:
            req = client_socket.read()
            if req:
                self.decode_path(req.decode("utf-8"))
                # self.verbose and print("Decoded path:", self.path)
                self.send_file(client_socket)
                client_socket.close()

                if self.query != "":
                    self.handle_query()
                return
        except OSError as e:
            if e.args[0] == 128:        # ENOTCONN
                pass
        except Exception as e:
            self.verbose and print("Err:", e)
            print_exception(e)
        client_socket.close()

    def client_handler(self, srv):
        try:
            client_socket, addr = srv.accept()
        except OSError as e:
            if e.args[0] == 23:         # Too many open files: we forgot to close a socket somewhere
                pass
        # self.verbose and print("Serving:", addr[0])
        self.whodat[addr[0]] = ("unknown", addr[0])
        client_socket.setblocking(False)
        client_socket.setsockopt(SOL_SOCKET, 20, self.req_handler)

def main():
    rgb[0] = (64, 0, 0)
    rgb.write()
    sleep(10)               # During development, so the chip allows file transfers
    con = Connect()
    con.verbose = True
    con.reconnect()

    rgb[0] = (64, 64, 0)
    rgb.write()
    mesh = Mesh()
    mesh.con = con
    mesh.verbose = True
    if con.sta_host_ip:
        # mesh.verbose and print(f"{w.name()}: Connected to {con.ssid} at {con.sta_host_ip}")
        mesh.add_neighbor(con.sta_host_ip, con.ssid, "unknown")

    rgb[0] = (0, 64, 0)
    rgb.write()

    while True:
        sleep(3 * 60)
        print("==============================================================")
        mesh.show_neighbors()
        mesh.colors()
        if con.sta and con.sta.isconnected() == False:
            con.reconnect()
            if con.sta_host_ip:
                # mesh.verbose and print(f"{w.name()}: Connected to {con.ssid} at {con.sta_host_ip}")
                mesh.add_neighbor(con.sta_host_ip, con.ssid, "unknown")

main()

The execute() method handles special message types the node can receive. One is a message that sets the colors and brightness of the RGB LED. This is handy for debugging, as not all nodes will have a computer attached for serial output.

It also handles printing out messages, and the reply to the greet() packet. The latter calls who_name() to associate the name with the IP addresses we got from the connecting node.

The whole point of a mesh network is to communicate with nodes we can’t connect to directly. That’s where the direct() method comes in. It looks at all the IP addresses we know about and returns False if the address given is unreachable.

We finally get to direct_msg(), the heart of the code. This assigns sequence numbers and inserts them into the payload, and then tries 5 times to send the packet, stopping at the first success. If it gets a reply, it adds the node as a neighbor (it lets add_neighbor() handle duplicates).

The broadcast() method is simple. It goes through the neighbor list and sends the packet to each one.

We use the HTTP GET method for packets, which means some packets might have a question mark (a ‘query’) with commands in it. If the command is “packet”, we collect the neighbor information, store it, and then check to see if the packet is for us. If it was, we call execute(). Otherwise we decrement the time_to_live number and broadcast the packet to everyone we know.

If the command was “text”, then we got it from a web browser (more on that in a bit). We repackage the data (adding our IP addresses, time_to_live, etc.) and broadcast it.

The part of the HTTP packet ahead of the query is the path. This is normally the file the server will read and send to the browser, and indeed we do that if asked. But we also handle “fake” files /text and /none. The /text file just acknowledges that we got a text message, so the web browser that sent it knows we got it. The /none file answers the greet message by returning our name, ssid, and IP addresses.

The bulk of the code in send_file() creates the /index.html file to send to any web browser that connects with us. This file prints out node information and includes a form for sending messages.

The form looks like this:

I named my nodes Alice, Bob, and Carol (the next one will be Ted).

We can see that Carol has Alice as a neighbor, but Carol can’t see Bob. I sent a message to Bob, telling Bob my browser was Joe, and Carol sent it to Alice who then sent it to Bob. The meshing worked (finally, after three weeks of coding and debugging).

The main() method sets the RGB LED to red, then connects to anyone it can, sets the RGB LED to yellow while it initializes the Mesh class, and then sets it to green before looping to do something every 3 minutes just to show that the mesh is meshing.

The something that it does is just to print out the neighbor list on the console (if there is one) and then broadcast a message to set everyone’s LED to a random color. Again, just to show the system is up and running.

When setting up a node out somewhere here on the farm, it is nice not to need a laptop to see what’s going on. Watching the LED change color tells me everything is working.

It is now easy to add features. You can collect temperature, humidity, and other weather data and send it out to the mesh. You can turn lights on and off, or tell when the well pump is running (you all have those right? Or is it just me?)

Each node has 150 megabits of bandwidth to play with (15 megabytes per second). Our typical packet length is about 100 bytes, so we could theoretically send 150,000 packets per second. We send each of our packets 3 times per node on the assumption all nodes are within 3 hops of one another, but we could easily change this, or make it an option in the web form. But the network would have to get pretty big before we ran out of bandwidth, even though we waste it by using the simple design.

Programming

Education

Software Development

Python