# Python Radio 51: A Peek Under The Covers

## Inside MicroPython to Build an SDR Radio

Simon Quellen Field

Follow

20 min read

·

Aug 26, 2025

Listen

Share

More

Press enter or click to view image in full size

MidJourney

In Python Radio 50, we built an entire AM radio in software. In Python, no less.

Python is not known for its speed. However, when we need speed, we often find that there are built-in functions that run as fast as the machine can go. These functions are written in C, the language that Python is written in.

In this chapter, we will build an SDR AM radio receiver (and transmitter) in MicroPython for the Raspberry Pi Pico 2 (the RP2350).

This $5 processor will replace both the laptop computer and the RTL-SDR USB dongle we have used in the past.

I knew going into this project that it was ambitious. What I did not know was that it would take me six weeks of effort. For you, it will only take a few minutes, as I will provide not only the working code but the script to build it.

The RP2350 has a 12-bit analog-to-digital converter (ADC) that we will connect to an antenna. It also features a pulse-width modulator (PWM) that we will connect to a speaker. We add a battery and the software, and we’re done.

Right away, we run into problems.

MicroPython on the RP2350 only supports reading one sample at a time from the ADC. This would be OK if we could call it millions of times in a second, but MicroPython is not up to that.

We have the same problem with the PWM. One sample at a time.

To solve both problems, we need to add high-speed DMA access to both peripherals to the MicroPython firmware. That means we need to write a Python module in C and link it in.

Adding simple C modules to MicroPython is easy. The developers have given us a nice mechanism for doing this. Unfortunately, ours is not a simple module. It needs to access subroutines from the Pico SDK and from the Arm CMSIS system. The easy mechanism can’t do that.

So we do it the hard way (something that in itself cost me three weeks of work). But I made a script that does all the work. You just fire and forget.

MicroPython is built under Linux. I have many Linux machines, but my big, beefy, fast server is running Windows. So I use Windows System for Linux to do the build. It runs Linux under Windows. You type this into a CMD window:

wsl -d Ubuntu

Now you are running a Linux shell. It’s that easy.

In my Linux home directory, I made two subdirectories, sdr_radio, and AM_sdr_radio_final. Then I executed the following shell script to build MicroPython with my sdr_radio.c module (placed in the sdr_radio directory):

#!/bin/bash
set -e # Exit immediately if any command fails

# --- Configuration ---
MPY_VERSION="v1.25.0"
BOARD="RPI_PICO2"
PROJECT_ROOT=~/AM_sdr_pico2_final
# This is the directory where you have staged all your working, vendored files.
USER_SOURCE_DIR=~/sdr_radio

# --- Sanity Check ---
if [ ! -d "${USER_SOURCE_DIR}" ]; then
    echo "Error: User source directory not found at ${USER_SOURCE_DIR}"
    exit 1
fi

echo "--- STARTING THE DEFINITIVE BUILD (MANUAL VENDOR + CORRECT PATHS) ---"

# --- STEPS 1-2: SETUP & VENDORING ---
echo "--- [1/5] Setting up project structure..."
rm -rf ${PROJECT_ROOT}
mkdir -p ${PROJECT_ROOT}

echo "--- [2/5] Cloning MicroPython and its core submodules..."
git clone --depth 1 -b ${MPY_VERSION} https://github.com/micropython/micropython.git ${PROJECT_ROOT}/micropython
cd ${PROJECT_ROOT}/micropython
git submodule update --init --recursive

# Add pico-extras, which is separate
git submodule add https://github.com/raspberrypi/pico-extras.git lib/pico-extras
git submodule update --init lib/pico-extras

# --- BRUTE-FORCE VENDORING of CMSIS ---
# The git submodule process is unreliable. We will download and place the files manually.
echo "Manually downloading and vendoring CMSIS-DSP library..."
# Create the target directory structure
mkdir -p ./lib/vendor/CMSIS_5
# Download a known-good version of the library as a ZIP file
wget -O cmsis.zip https://github.com/ARM-software/CMSIS_5/archive/refs/tags/5.9.0.zip
# Unzip it into a temporary directory
unzip -q cmsis.zip -d ./lib/vendor/
# Move the contents into our final location
mv ./lib/vendor/CMSIS_5-5.9.0/* ./lib/vendor/CMSIS_5/
# Clean up
rm cmsis.zip
rm -rf ./lib/vendor/CMSIS_5-5.9.0/

# --- VERIFICATION STEP ---
# Check the path where we downloaded the files.
ARM_MATH_PATH="./lib/vendor/CMSIS_5/CMSIS/DSP/Include/arm_math.h"
echo "Verifying that arm_math.h exists at ${ARM_MATH_PATH}..."
if [ -f "$ARM_MATH_PATH" ]; then
    echo "SUCCESS: arm_math.h found in vendored directory."
else
    echo "FATAL ERROR: arm_math.h was NOT found after manual download."
    exit 1
fi

# --- STEP 3: CREATE THE SELF-CONTAINED SDR MODULE ---
echo "--- [3/5] Creating sdr_radio module and copying all required sources... ---"
MODULE_PATH=./extmod/sdr_radio
mkdir -p ${MODULE_PATH}
echo "Copying your staged module files from ${USER_SOURCE_DIR}..."
cp ${USER_SOURCE_DIR}/* ${MODULE_PATH}/

# --- STEP 4: MODIFY BUILD FILES (THE DEFINITIVE FIX) ---
echo "--- [4/5] Configuring the MicroPython build... ---"
cd ./ports/rp2

# 1. Reset and activate the module in the C preprocessor.
cp mpconfigport.h.orig mpconfigport.h 2>/dev/null || cp mpconfigport.h mpconfigport.h.orig
echo "" >> mpconfigport.h
echo "// Enable the custom sdr_radio module" >> mpconfigport.h
echo "#define MICROPY_PY_SDR_RADIO (1)" >> mpconfigport.h

# 2. Reset and inject the complete module configuration into CMake.
cp CMakeLists.txt.orig CMakeLists.txt 2>/dev/null || cp CMakeLists.txt CMakeLists.txt.orig
TARGET_LINE_SOURCES="set(PICO_SDK_COMPONENTS"
CUSTOM_BLOCK_SOURCES="\
\n# --- Customization for sdr_radio module ---\n\
# We will now build the CMSIS-DSP sources directly into the firmware.\n\
\n\
# Part 1: Add all necessary include paths.\n\
include_directories(\n\
    \${MICROPY_DIR}/extmod/sdr_radio \n\
    \${MICROPY_DIR}/py \n\
    \${MICROPY_DIR}/ports/rp2 \n\
    # Include paths for CMSIS-DSP Public API, Private Helpers, and Core types.\n\
    \${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Include \n\
    \${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/PrivateInclude \n\
    \${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/Core/Include \n\
    # Your other existing include paths\n\
    \${MICROPY_DIR}/lib/pico-extras/src/rp2_common/pico_audio_i2s/include \n\
    \${MICROPY_DIR}/lib/pico-extras/src/common/pico_audio/include \n\
    \${MICROPY_DIR}/lib/pico-extras/src/common/pico_util_buffer/include \n\
)\n\
\n\
# \n\
# Part 2: Create a list containing ONLY the main 'roll-up' source files.\n\
set(CMSIS_DSP_SOURCES\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/BasicMathFunctions/BasicMathFunctions.c\"\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/CommonTables/CommonTables.c\"\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/ComplexMathFunctions/ComplexMathFunctions.c\"\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/ControllerFunctions/ControllerFunctions.c\"\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/FastMathFunctions/FastMathFunctions.c\"\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/FilteringFunctions/FilteringFunctions.c\"\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/MatrixFunctions/MatrixFunctions.c\"\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/StatisticsFunctions/StatisticsFunctions.c\"\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/SupportFunctions/SupportFunctions.c\"\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/TransformFunctions/TransformFunctions.c\"\n\
    \"\${MICROPY_DIR}/lib/vendor/CMSIS_5/CMSIS/DSP/Source/QuaternionMathFunctions/QuaternionMathFunctions.c\"\n\
)\n\
\n\
# Part 3: Add our module's C file AND the curated CMSIS-DSP SOURCE FILES to MicroPython's build list.\n\
list(APPEND MICROPY_SOURCE_PORT \n\
    \${MICROPY_DIR}/extmod/sdr_radio/sdr_radio.c\n\
    \${CMSIS_DSP_SOURCES}\n\
)\n\
list(APPEND MICROPY_SOURCE_QSTR \${MICROPY_DIR}/extmod/sdr_radio/sdr_radio.c)\n\
\n\
# --- End of customizations ---\n"
awk -v block="$CUSTOM_BLOCK_SOURCES" -v target="$TARGET_LINE_SOURCES" 'index($0, target) {print block} 1' CMakeLists.txt > CMakeLists.txt.new && mv CMakeLists.txt.new CMakeLists.txt

### sed -i '/Execute _boot.py to set up the filesystem/a \        mp_printf(MP_PYTHON_PRINTER, "Kilroy with Micropython threads and ADC fix\\n");' main.c 

# --- STEP 5: BUILD THE FIRMWARE ---
echo "--- [5/5] Starting the final MicroPython build ---"
make -j4 BOARD=${BOARD}

echo ""
echo "--- BUILD SUCCESSFUL! ---"
echo "Firmware is at: ${PROJECT_ROOT}/micropython/ports/rp2/build-${BOARD}/firmware.uf2"
ls -l build-${BOARD}/firmware.uf2
cp build-${BOARD}/firmware.uf2 /mnt/c/simon/sdr_radio_pico

echo "--- VERIFYING MODULE PRESENCE IN SYMBOL TABLE ---"
# Check the final ELF for the module symbol. This will now pass.
if arm-none-eabi-nm "build-${BOARD}/firmware.elf" | grep -q "sdr_radio_user_cmodule"; then
    echo "SUCCESS: sdr_radio module symbol found in the firmware."
else
    echo "ERROR: sdr_radio module symbol was NOT found in the firmware."
    exit 1
fi

echo ""
echo "--- ALL STEPS COMPLETE. The module will now be visible in the REPL. ---"

Whew!

Now we have a file called firmware.uf2. We hold down the little button on the RP2350, cycle power, and it’s in boot mode, and shows up in Windows as a new disk drive. We copy firmware.uf2 into that new directory, and the microcomputer boots the new firmware.

Of course, before building it, we need our new module:

#include "py/runtime.h"
#include "py/mphal.h"
#include <math.h>
#include <string.h>
#include "hardware/dma.h"
#include "hardware/adc.h"
#include "hardware/irq.h"
#include "hardware/sync.h"
#include "hardware/resets.h"
#include <float.h>
#include "hardware/clocks.h"
#include "hardware/pwm.h"
#include "arm_math.h"
#include "pico/multicore.h"

#define ADC_SAMPLE_RATE 500000
#define AUDIO_SAMPLE_RATE 22050

#define mult_q31(a, b) ((q31_t)(((int64_t)(a) * (b)) >> 31))

typedef struct _sdr_radio_obj_t {
    mp_obj_base_t base;
    uint32_t tune_freq_hz;

    q31_t nco_phase;             // Current phase accumulator
    q31_t nco_phase_increment;   // Phase step per sample

    // --- State for the Iterative NCO (Mixer) ---
    q31_t nco_i;            // Current I value (cos) of the NCO, Q31 format
    q31_t nco_q;            // Current Q value (sin) of the NCO, Q31 format
    q31_t nco_cos_inc;      // Pre-calculated cos(phase_increment)
    q31_t nco_sin_inc;      // Pre-calculated sin(phase_increment)

    // --- State for the fixed-point RF DC Blocker ---
    q31_t dc_block_i_x1;
    q31_t dc_block_i_y1;
    q31_t dc_block_q_x1;
    q31_t dc_block_q_y1;

    // --- State for the LPF (Cascaded EMA) ---
    q31_t ema_i_s1, ema_i_s2, ema_i_s3;
    q31_t ema_q_s1, ema_q_s2, ema_q_s3;;
    
 q31_t demod_mag_x1;

    // --- State for the Audio HPF (DC Blocker) ---
    q31_t audio_hpf_x1;
 q31_t audio_hpf_y1;

 q31_t agc_smoothed_peak;

 q31_t audio_ema_lpf;

 bool is_am_mode;

    q31_t bfo_phase;
    q31_t bfo_phase_increment;

 ///////////////////////////////////////////////////////////////
 ////////////// Transmitter Section ////////////////////////////
 ///////////////////////////////////////////////////////////////
    uint32_t tx_carrier_freq_hz;
    q31_t tx_nco_phase;
    q31_t tx_nco_phase_increment;
    float32_t tx_modulation_index;

    uint32_t capture_sample_rate;
    uint32_t capture_num_samples;
    uint32_t adc_clkdiv;

} sdr_radio_obj_t;


// The internal C buffers that the DMA will write to.
// The size MUST match the buffer size used in the Python script.
#define MAX_CAPTURE_BUFFER_SIZE 8192

static int adc_dma_chan_A = -1;
static int adc_dma_chan_B = -1;

// Internal ping-pong buffers for the DMA
static uint32_t capture_buf_A[MAX_CAPTURE_BUFFER_SIZE];
static uint32_t capture_buf_B[MAX_CAPTURE_BUFFER_SIZE];



// Helper function to guarantee a clean state
static void reset_sdr_state(sdr_radio_obj_t *self) {
    self->nco_phase = 0;
    self->dc_block_i_x1 = 0;
    self->dc_block_i_y1 = 0;
    self->dc_block_q_x1 = 0;
    self->dc_block_q_y1 = 0;

 self->ema_i_s1=0; self->ema_i_s2=0; self->ema_i_s3=0;
 self->ema_q_s1=0; self->ema_q_s2=0; self->ema_q_s3=0;

    self->agc_smoothed_peak = 1000;

    // Initialize the Audio HPF state
    self->demod_mag_x1 = 0;
    self->audio_hpf_y1 = 0;

 self->bfo_phase = 0;

 self->audio_ema_lpf = 0;
}

// Exposed to Python to make tests deterministic
static mp_obj_t sdr_radio_reset_state(mp_obj_t self_in) {
    sdr_radio_obj_t *self = MP_OBJ_TO_PTR(self_in);
    reset_sdr_state(self);
    return mp_const_none;
}
static MP_DEFINE_CONST_FUN_OBJ_1(sdr_radio_reset_state_obj, sdr_radio_reset_state);


static mp_obj_t sdr_radio_set_mode(mp_obj_t self_in, mp_obj_t is_am_obj) {
    sdr_radio_obj_t *self = MP_OBJ_TO_PTR(self_in);
 self->is_am_mode = mp_obj_is_true(is_am_obj);
 
    return mp_const_none;
}
static MP_DEFINE_CONST_FUN_OBJ_2(sdr_radio_set_mode_obj, sdr_radio_set_mode);



static mp_obj_t sdr_radio_make_new(const mp_obj_type_t *type, size_t n_args, size_t n_kw, const mp_obj_t *args) {

    sdr_radio_obj_t *self = mp_obj_malloc(sdr_radio_obj_t, type);

    reset_sdr_state(self);
 self->bfo_phase = 0;
    self->nco_phase_increment = (uint32_t)( ( (uint64_t)self->tune_freq_hz << 32 ) / ADC_SAMPLE_RATE );
    self->capture_sample_rate = 0;
    self->capture_num_samples = 0;

    return MP_OBJ_FROM_PTR(self);
}


static mp_obj_t sdr_radio_tune(mp_obj_t self_in, mp_obj_t freq_obj) {
    sdr_radio_obj_t *self = MP_OBJ_TO_PTR(self_in);
    
    // 1. Get the desired station frequency (e.g., 810000) from Python.
    uint32_t station_freq_hz = mp_obj_get_int(freq_obj);

    // --- Alias Calculation ---
    // This logic calculates the NCO frequency needed to tune to a station
    // by using undersampling (aliasing) to bring it into the first Nyquist zone.
    
    // Find the remainder when the station frequency is divided by the sample rate.
    uint32_t remainder = station_freq_hz % ADC_SAMPLE_RATE;

    uint32_t nco_tune_freq_hz;
    
    // Check which half of the Nyquist zone the remainder falls into.
    if (remainder < (ADC_SAMPLE_RATE / 2)) {
        // If it's in the lower half, the alias appears directly.
        // e.g., for a 190kHz station, remainder is 190k. We tune to 190k.
        nco_tune_freq_hz = remainder;
    } else {
        // If it's in the upper half, the alias is mirrored from the top.
        // e.g., for an 810kHz station, remainder is 310k. We tune to 500k-310k = 190k.
        nco_tune_freq_hz = ADC_SAMPLE_RATE - remainder;
    }

    // Store the calculated NCO frequency in our object.
    self->tune_freq_hz = nco_tune_freq_hz;
    
    // Recalculate the NCO phase increment with the new frequency.
    self->nco_phase_increment = (q31_t)(((uint64_t)self->tune_freq_hz << 31) / ADC_SAMPLE_RATE);

    return mp_const_none;
}


static MP_DEFINE_CONST_FUN_OBJ_2(sdr_radio_tune_obj, sdr_radio_tune);




static mp_obj_t fast_sdr_pipeline(mp_obj_t self_in, mp_obj_t args_in) {

    sdr_radio_obj_t *self = MP_OBJ_TO_PTR(self_in);

    size_t n_args;
    mp_obj_t *args;
    mp_obj_get_array(args_in, &n_args, &args);

    if (n_args < 3) {
        mp_raise_TypeError(MP_ERROR_TEXT("Requires at least adc, out, and scratch buffers"));
    }
    mp_buffer_info_t adc_info;     mp_get_buffer_raise(args[0], &adc_info,     MP_BUFFER_READ);
    mp_buffer_info_t out_info;     mp_get_buffer_raise(args[1], &out_info,     MP_BUFFER_WRITE);
    mp_buffer_info_t scratch_info; mp_get_buffer_raise(args[2], &scratch_info, MP_BUFFER_WRITE);

    // --- Buffer Pointers and Sizes ---
    uint16_t *adc_in_ptr = (uint16_t *)adc_info.buf;
    uint32_t *pwm_out_ptr = (uint32_t *)out_info.buf;
    const int num_adc_samples = adc_info.len / sizeof(uint16_t);
    const int num_audio_samples = out_info.len / sizeof(uint32_t);

    // --- DSP Constants ---
    const q31_t DC_BLOCK_R = 0x7F800000;
    const q31_t RF_LPF_ALPHA = 0x20000000; // Alpha=0.25, wide ~20kHz RF LPF
    const q31_t RF_LPF_ONE_MINUS_ALPHA = 0x7FFFFFFF - RF_LPF_ALPHA;
    const int DECIMATION_FACTOR = ADC_SAMPLE_RATE / 22050;
    const q31_t AUDIO_HPF_R = 0x7E000000; // ~112 Hz HPF cutoff

    q31_t *temp_audio_buf = (q31_t*)scratch_info.buf;
    int audio_idx = 0;

 int decimation_counter = 0;
 q31_t i_filtered = 0;
 q31_t q_filtered = 0;

    if (self->is_am_mode) {
        // ====================================================================
        //  FAST PATH for AM MODE (No RF DC Blocker)
        // ====================================================================
        for (int i = 0; i < num_adc_samples; i++) {
            q31_t sample = ((q31_t)adc_in_ptr[i] - 2048) << 19;

            q31_t nco_s = arm_sin_q31(self->nco_phase);
            q31_t nco_c = arm_cos_q31(self->nco_phase);

            self->nco_phase += self->nco_phase_increment;
            q31_t i_raw = mult_q31(sample, nco_c);
            q31_t q_raw = mult_q31(sample, nco_s); // Use positive sine for Q

            // 3-Stage Cascaded EMA Low-Pass Filter
            q31_t i_s1_out = mult_q31(self->ema_i_s1, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(i_raw, RF_LPF_ALPHA);
            self->ema_i_s1 = i_s1_out;
            q31_t i_s2_out = mult_q31(self->ema_i_s2, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(i_s1_out, RF_LPF_ALPHA);
            self->ema_i_s2 = i_s2_out;
            // q31_t i_filtered = mult_q31(self->ema_i_s3, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(i_s2_out, RF_LPF_ALPHA);
            i_filtered = mult_q31(self->ema_i_s3, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(i_s2_out, RF_LPF_ALPHA);
            self->ema_i_s3 = i_filtered;

            q31_t q_s1_out = mult_q31(self->ema_q_s1, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(q_raw, RF_LPF_ALPHA);
            self->ema_q_s1 = q_s1_out;
            q31_t q_s2_out = mult_q31(self->ema_q_s2, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(q_s1_out, RF_LPF_ALPHA);
            self->ema_q_s2 = q_s2_out;
            // q31_t q_filtered = mult_q31(self->ema_q_s3, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(q_s2_out, RF_LPF_ALPHA);
            q_filtered = mult_q31(self->ema_q_s3, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(q_s2_out, RF_LPF_ALPHA);
            self->ema_q_s3 = q_filtered;
            
            // Decimation and Audio Path
   if (++decimation_counter >= DECIMATION_FACTOR) {
                decimation_counter = 0;
                if (audio_idx < num_audio_samples) {
     // --- AM Demodulation (Fast Approximation) ---
                    q31_t abs_i = (i_filtered > 0) ? i_filtered : -i_filtered;
                    q31_t abs_q = (q_filtered > 0) ? q_filtered : -q_filtered;
     q31_t max_val, min_val;
     if (abs_i > abs_q) {
      max_val = abs_i;
      min_val = abs_q;
     } else {
      max_val = abs_q;
      min_val = abs_i;
     }

     // Magnitude ≈ max + 0.25*min
     q31_t magnitude = __QADD(max_val, min_val >> 2);
     q31_t demodulated_signal = magnitude;
     
     // Audio HPF
     q31_t diff = __QSUB(demodulated_signal, self->audio_hpf_x1);
     q31_t sum = __QADD(self->audio_hpf_y1, diff);
     q31_t audio_sample = mult_q31(AUDIO_HPF_R, sum);
     self->audio_hpf_x1 = magnitude;
     self->audio_hpf_y1 = audio_sample;
     
     temp_audio_buf[audio_idx++] = audio_sample;
    }
            }
        }
    } else {
        // ====================================================================
        //  FAST PATH for CW/SSB MODE (with BFO)
        // ====================================================================
        for (int i = 0; i < num_adc_samples; i++) {
            // Step 1: ADC Scaling
            q31_t sample = ((q31_t)adc_in_ptr[i] - 2048) << 19;

            // Step 2: NCO & Mixer
            q31_t nco_s = arm_sin_q31(self->nco_phase);
            q31_t nco_c = arm_cos_q31(self->nco_phase);

            self->nco_phase += self->nco_phase_increment;
            q31_t i_raw = mult_q31(sample, nco_c);
            q31_t q_raw = mult_q31(sample, nco_s);

            // Step 3: RF DC Blocker
   q31_t i_blocked = i_raw - self->dc_block_i_x1 + mult_q31(DC_BLOCK_R, self->dc_block_i_y1);
   self->dc_block_i_x1 = i_raw; self->dc_block_i_y1 = i_blocked;
   q31_t q_blocked = q_raw - self->dc_block_q_x1 + mult_q31(DC_BLOCK_R, self->dc_block_q_y1);
   self->dc_block_q_x1 = q_raw; self->dc_block_q_y1 = q_blocked;

            // 3-Stage Cascaded EMA Low-Pass Filter
            q31_t i_s1_out = mult_q31(self->ema_i_s1, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(i_blocked, RF_LPF_ALPHA);
            self->ema_i_s1 = i_s1_out;
            q31_t i_s2_out = mult_q31(self->ema_i_s2, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(i_s1_out, RF_LPF_ALPHA);
            self->ema_i_s2 = i_s2_out;
            // q31_t i_filtered = mult_q31(self->ema_i_s3, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(i_s2_out, RF_LPF_ALPHA);
            i_filtered = mult_q31(self->ema_i_s3, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(i_s2_out, RF_LPF_ALPHA);
            self->ema_i_s3 = i_filtered;

            q31_t q_s1_out = mult_q31(self->ema_q_s1, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(q_blocked, RF_LPF_ALPHA);
            self->ema_q_s1 = q_s1_out;
            q31_t q_s2_out = mult_q31(self->ema_q_s2, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(q_s1_out, RF_LPF_ALPHA);
            self->ema_q_s2 = q_s2_out;
            // q31_t q_filtered = mult_q31(self->ema_q_s3, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(q_s2_out, RF_LPF_ALPHA);
            q_filtered = mult_q31(self->ema_q_s3, RF_LPF_ONE_MINUS_ALPHA) + mult_q31(q_s2_out, RF_LPF_ALPHA);
            self->ema_q_s3 = q_filtered;

            // Step 5: Decimation and Audio Path
            if (++decimation_counter >= DECIMATION_FACTOR) {
                decimation_counter = 0;
                if (audio_idx < num_audio_samples) {
                    // Step 6: BFO Mixing for CW/SSB Demodulation
                    q31_t bfo_c = arm_cos_q31(self->bfo_phase);
                    q31_t bfo_s = arm_sin_q31(self->bfo_phase);
                    self->bfo_phase += self->bfo_phase_increment;

                    // This is a complex multiplication that shifts the signal by the BFO frequency.
                    // For SSB, this is single-sideband demodulation.
                    // For CW, this shifts the 0 Hz DC signal up to the audible BFO frequency.
                    q31_t demodulated_signal = mult_q31(i_filtered, bfo_c) - mult_q31(q_filtered, bfo_s);

                    // Step 7: Audio HPF to remove any remaining DC
                    q31_t diff = __QSUB(demodulated_signal, self->audio_hpf_x1);
                    q31_t sum = __QADD(self->audio_hpf_y1, diff);
                    q31_t audio_sample = mult_q31(AUDIO_HPF_R, sum);
                    self->audio_hpf_x1 = demodulated_signal;
                    self->audio_hpf_y1 = audio_sample;
                    
                    temp_audio_buf[audio_idx++] = audio_sample;
                }
            }
        }
    }
    // =============================================================
    //  Sample-by-Sample AGC (Common to both paths)
    // =============================================================
    const q31_t AGC_ATTACK_ALPHA = 0x01000000;
    const q31_t AGC_DECAY_ALPHA  = 0x00100000;
    for (int i = 0; i < audio_idx; i++) {
        q31_t current_sample = temp_audio_buf[i];
        q31_t current_abs = (current_sample > 0) ? current_sample : -current_sample;

        if (current_abs > self->agc_smoothed_peak) {
            self->agc_smoothed_peak = mult_q31(self->agc_smoothed_peak, (0x7FFFFFFF - AGC_ATTACK_ALPHA)) + mult_q31(current_abs, AGC_ATTACK_ALPHA);
        } else {
            self->agc_smoothed_peak = mult_q31(self->agc_smoothed_peak, (0x7FFFFFFF - AGC_DECAY_ALPHA)) + mult_q31(current_abs, AGC_DECAY_ALPHA);
        }

        int32_t gain_shifts = 0;
        if (self->agc_smoothed_peak > 1000) {
             gain_shifts = __builtin_clz(self->agc_smoothed_peak) - 2;
        }
        if (gain_shifts < 0) gain_shifts = 0;
        
        q31_t final_audio;
        if (gain_shifts > 0) {
            final_audio = __SSAT(((int64_t)current_sample << gain_shifts), 32);
        } else {
            final_audio = current_sample;
        }
        
        int32_t scaled_sample = (final_audio >> 23) + 128;
        if (scaled_sample > 255) scaled_sample = 255; else if (scaled_sample < 0) scaled_sample = 0;
        
        pwm_out_ptr[i] = (128 << 16) | (uint32_t)scaled_sample;
    }

    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_2(fast_sdr_pipeline_obj, fast_sdr_pipeline);




static mp_obj_t sdr_radio_set_bfo(mp_obj_t self_in, mp_obj_t freq_obj) {
    sdr_radio_obj_t *self = MP_OBJ_TO_PTR(self_in);
    
    // Get the frequency as an integer from the Python object
    int bfo_freq_hz = mp_obj_get_int(freq_obj);

    // Calculate the phase increment for the BFO.
    // NOTE: This calculation uses AUDIO_SAMPLE_RATE because the BFO
    // operates on the decimated, audio-rate signal.
    // It also uses "<< 31" because arm_cos_q31 expects a signed Q31 input.
    self->bfo_phase_increment = (q31_t)(((uint64_t)bfo_freq_hz << 31) / AUDIO_SAMPLE_RATE);

    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_2(sdr_radio_set_bfo_obj, sdr_radio_set_bfo);




static bool consumer_wants_buffer_A = true;
uint32_t sum_a, sum_b;

static mp_obj_t sdr_radio_capture_chunk(mp_obj_t self_in, mp_obj_t buf_obj) {

    sdr_radio_obj_t *self = MP_OBJ_TO_PTR(self_in);

    mp_buffer_info_t bufinfo;
    mp_get_buffer_raise(buf_obj, &bufinfo, MP_BUFFER_WRITE);

    uint32_t *src_buf_to_copy = NULL;

    if (consumer_wants_buffer_A) {
        while (dma_channel_hw_addr(adc_dma_chan_A)->transfer_count > 0) {
            // Busy-wait
        }
        dma_channel_acknowledge_irq0(adc_dma_chan_A);
        src_buf_to_copy = capture_buf_A;
        
        dma_channel_set_write_addr(adc_dma_chan_B, capture_buf_B, true); // true = trigger now
    } else {
        while (dma_channel_hw_addr(adc_dma_chan_B)->transfer_count > 0) {
            // Busy-wait
        }
        dma_channel_acknowledge_irq0(adc_dma_chan_B);
        src_buf_to_copy = capture_buf_B;
        
        dma_channel_set_write_addr(adc_dma_chan_A, capture_buf_A, true); // true = trigger now
    }

    consumer_wants_buffer_A = !consumer_wants_buffer_A;

 uint32_t *dma_src = (uint32_t *)src_buf_to_copy;
 uint16_t *py_dest = (uint16_t *)bufinfo.buf;
  for (uint32_t i = 0; i < (self->capture_num_samples); ++i) {
  py_dest[i] = dma_src[i] & 0xFFFF;
 }

    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_2(sdr_radio_capture_chunk_obj, sdr_radio_capture_chunk);




static mp_obj_t sdr_radio_deinit_capture() {
    // Check if channels were claimed before trying to use them
    if (adc_dma_chan_A != -1) {
        dma_channel_abort(adc_dma_chan_A);
        dma_channel_unclaim(adc_dma_chan_A);
    }
    
    if (adc_dma_chan_B != -1) {
        dma_channel_abort(adc_dma_chan_B);
        dma_channel_unclaim(adc_dma_chan_B);
    }
    
    adc_run(false);
    
    adc_dma_chan_A = -1;
    adc_dma_chan_B = -1;

    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_0(sdr_radio_deinit_capture_obj, sdr_radio_deinit_capture);


// =========================================================================
// 1. THE PWM OBJECT DEFINITION
// =========================================================================
typedef struct _machine_pwm_obj_t {
    mp_obj_base_t base;
    uint8_t slice;
    uint8_t channel;
    uint8_t invert;
    uint8_t duty_type;
    mp_int_t duty;
    bool is_streaming;
    int stream_dma_chan;
} machine_pwm_obj_t;

// Our own state for the DMA channel.
static bool audio_is_configured = false;




static mp_obj_t sdr_radio_configure_and_init_capture(mp_obj_t self_in, mp_obj_t rate_obj, mp_obj_t size_obj) {

    sdr_radio_obj_t *self = MP_OBJ_TO_PTR(self_in);

    self->capture_sample_rate = mp_obj_get_int(rate_obj);
    self->capture_num_samples = mp_obj_get_int(size_obj);

    reset_block(RESETS_RESET_ADC_BITS | RESETS_RESET_DMA_BITS);
    unreset_block_wait(RESETS_RESET_ADC_BITS | RESETS_RESET_DMA_BITS);

    adc_init();
    adc_gpio_init(26);
    adc_select_input(0);

    adc_fifo_setup(true, true, 1, false, false);

    float div = 48000000.0f / (float)self->capture_sample_rate;
    adc_set_clkdiv(div);

 uint32_t save_em[6];
    for (int i = 0; i < 6; ++i) {
  save_em[i] = dma_claim_unused_channel(true);
 }

    adc_dma_chan_A = dma_claim_unused_channel(true);
    adc_dma_chan_B = dma_claim_unused_channel(true);

    for (int i = 0; i < 6; ++i) {
        dma_channel_unclaim(save_em[i]);
 }

    dma_channel_config cA = dma_channel_get_default_config(adc_dma_chan_A);
    channel_config_set_transfer_data_size(&cA, DMA_SIZE_32);
    channel_config_set_read_increment(&cA, false);
    channel_config_set_write_increment(&cA, true);
    channel_config_set_dreq(&cA, DREQ_ADC);
    channel_config_set_irq_quiet(&cA, false);
    dma_channel_configure(adc_dma_chan_A, &cA, capture_buf_A, &adc_hw->fifo, self->capture_num_samples, false);
    mp_hal_delay_ms(1);

    dma_channel_config cB = dma_channel_get_default_config(adc_dma_chan_B);
    channel_config_set_transfer_data_size(&cB, DMA_SIZE_32);
    channel_config_set_read_increment(&cB, false);
    channel_config_set_write_increment(&cB, true);
    channel_config_set_dreq(&cB, DREQ_ADC);
    channel_config_set_irq_quiet(&cB, false);
    dma_channel_configure(adc_dma_chan_B, &cB, capture_buf_B, &adc_hw->fifo, self->capture_num_samples, false);
    mp_hal_delay_ms(1);

    mp_hal_delay_ms(1);
    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_3(sdr_radio_configure_and_init_capture_obj, sdr_radio_configure_and_init_capture);




static mp_obj_t audio_configure(mp_obj_t pwm_obj, mp_obj_t sample_rate_obj) {

    machine_pwm_obj_t *pwm = MP_OBJ_TO_PTR(pwm_obj);

    if (!audio_is_configured) {
        pwm->stream_dma_chan = -1;
    }

    mp_int_t sample_rate = mp_obj_get_int(sample_rate_obj);
    
    // Configure PWM slice basics
    pwm_set_enabled(pwm->slice, false);
    pwm_set_wrap(pwm->slice, 255);
    uint32_t source_hz = clock_get_hz(clk_sys);
    float div = (float)source_hz / (256.0f * (float)sample_rate);
    if (div < 1.0f) div = 1.0f;
    pwm_set_clkdiv(pwm->slice, div);

    // Enable the PWM to send DREQ signals to the DMA
    hw_set_bits(&pwm_hw->slice[pwm->slice].csr, 1 << 3); // Set DMAEN bit

    // Set initial level and enable the PWM
    pwm_set_both_levels(pwm->slice, 128, 128);
    pwm_set_enabled(pwm->slice, true);

    if (pwm->stream_dma_chan < 0) {
        pwm->stream_dma_chan = dma_claim_unused_channel(true);
        if (pwm->stream_dma_chan < 0) {
             mp_raise_msg(&mp_type_RuntimeError, MP_ERROR_TEXT("Failed to claim a DMA channel for audio"));
        }
    }
    
    dma_channel_config c = dma_channel_get_default_config(pwm->stream_dma_chan);
    channel_config_set_transfer_data_size(&c, DMA_SIZE_32);
    channel_config_set_read_increment(&c, true);
    channel_config_set_write_increment(&c, false);
    channel_config_set_dreq(&c, pwm_get_dreq(pwm->slice));

    dma_channel_configure(
        pwm->stream_dma_chan, &c,
        &pwm_hw->slice[pwm->slice].cc, 
        NULL, // Source address will be set by audio_play_chunk
        0,    // Transfer count will be set by audio_play_chunk
        false // Do not trigger now
    );

    audio_is_configured = true;

    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_2(audio_configure_obj, audio_configure);




static mp_obj_t audio_play_chunk(mp_obj_t pwm_obj, mp_obj_t buf_obj) {

    machine_pwm_obj_t *pwm = MP_OBJ_TO_PTR(pwm_obj);

    if (!audio_is_configured || pwm->stream_dma_chan < 0) {
        mp_raise_msg(&mp_type_RuntimeError, MP_ERROR_TEXT("Audio not configured or DMA channel not claimed"));
    }

    mp_buffer_info_t bufinfo;
    mp_get_buffer_raise(buf_obj, &bufinfo, MP_BUFFER_READ);
    if (bufinfo.typecode != 'L') {
        mp_raise_ValueError(MP_ERROR_TEXT("Buffer must be of typecode 'L'."));
    }

    dma_channel_abort(pwm->stream_dma_chan);

    // 2. Get a clean, default configuration block.
    dma_channel_config c = dma_channel_get_default_config(pwm->stream_dma_chan);
    
    // 3. Re-populate the entire configuration.
    channel_config_set_transfer_data_size(&c, DMA_SIZE_32);
    channel_config_set_read_increment(&c, true);
    channel_config_set_write_increment(&c, false);
    channel_config_set_dreq(&c, pwm_get_dreq(pwm->slice));

    // 4. Atomically apply the full configuration and trigger the transfer.
    dma_channel_configure(
        pwm->stream_dma_chan,
        &c,
        &pwm_hw->slice[pwm->slice].cc, // Write address
        bufinfo.buf,                   // Read address (the new buffer)
        bufinfo.len / 4,               // Transfer count
        true                           // Trigger immediately
    );

    dma_channel_set_read_addr(pwm->stream_dma_chan, bufinfo.buf, false);
    dma_channel_set_trans_count(pwm->stream_dma_chan, bufinfo.len / 4, true); // true = trigger now

    pwm->is_streaming = true;
    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_2(audio_play_chunk_obj, audio_play_chunk);




static mp_obj_t audio_wait_done(mp_obj_t pwm_obj) {

    machine_pwm_obj_t *pwm = MP_OBJ_TO_PTR(pwm_obj);

    if (pwm->stream_dma_chan >= 0 && dma_channel_is_busy(pwm->stream_dma_chan)) {
        dma_channel_wait_for_finish_blocking(pwm->stream_dma_chan);
    }

    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_1(audio_wait_done_obj, audio_wait_done);




static mp_obj_t audio_deinit(mp_obj_t pwm_obj) {
    machine_pwm_obj_t *pwm = MP_OBJ_TO_PTR(pwm_obj);
    
    // Wait for any final transfer to complete.
    if (pwm->stream_dma_chan >= 0) {
        dma_channel_wait_for_finish_blocking(pwm->stream_dma_chan);
    }
    
    // Unclaim the channel ONLY when we are finished ---
    if (pwm->stream_dma_chan >= 0) {
        dma_channel_unclaim(pwm->stream_dma_chan);
        pwm->stream_dma_chan = -1;
    }

    if (pwm->is_streaming) {
        pwm_set_chan_level(pwm->slice, pwm->channel, 128); // Set to silence
        pwm->is_streaming = false;
    }
    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_1(audio_deinit_obj, audio_deinit);




static mp_obj_t sdr_radio_start_capture(mp_obj_t self_in) {
    sdr_radio_obj_t *self = MP_OBJ_TO_PTR(self_in);
 (void)self;

    adc_fifo_drain();
    dma_start_channel_mask(1u << adc_dma_chan_A);
    adc_run(true);

    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_1(sdr_radio_start_capture_obj, sdr_radio_start_capture);




static mp_obj_t configure_transmitter_pwm(mp_obj_t pwm_obj, mp_obj_t update_rate_obj, mp_obj_t top_obj) {
    machine_pwm_obj_t *pwm = MP_OBJ_TO_PTR(pwm_obj);
    uint32_t update_rate = mp_obj_get_int(update_rate_obj);
    uint32_t top = mp_obj_get_int(top_obj);

    pwm_set_enabled(pwm->slice, false);

    uint32_t source_hz = clock_get_hz(clk_sys);
    float div = (float)source_hz / ((float)(top + 1) * (float)update_rate);
    if (div < 1.0f) div = 1.0f;
    
    pwm_set_clkdiv(pwm->slice, div);
    pwm_set_wrap(pwm->slice, top);

    pwm_set_chan_level(pwm->slice, pwm->channel, 0);
    pwm_set_enabled(pwm->slice, true);

    audio_is_configured = true; 
    return mp_const_none;
}
static MP_DEFINE_CONST_FUN_OBJ_3(configure_transmitter_pwm_obj, configure_transmitter_pwm);


static mp_obj_t sdr_radio_set_tx_carrier(mp_obj_t self_in, mp_obj_t freq_obj, mp_obj_t pwm_rate_obj) {
    sdr_radio_obj_t *self = MP_OBJ_TO_PTR(self_in);
    self->tx_carrier_freq_hz = mp_obj_get_int(freq_obj);
    uint32_t pwm_update_rate = mp_obj_get_int(pwm_rate_obj);

    self->tx_nco_phase_increment = (q31_t)(((uint64_t)self->tx_carrier_freq_hz << 31) / pwm_update_rate);

    return mp_const_none;
}

static MP_DEFINE_CONST_FUN_OBJ_3(sdr_radio_set_tx_carrier_obj, sdr_radio_set_tx_carrier);


static mp_obj_t sdr_radio_am_transmit_pipeline(mp_obj_t self_in, mp_obj_t audio_buf_obj, mp_obj_t pwm_buf_obj) {
    sdr_radio_obj_t *self = MP_OBJ_TO_PTR(self_in);

    mp_buffer_info_t audio_info; mp_get_buffer_raise(audio_buf_obj, &audio_info, MP_BUFFER_READ);
    mp_buffer_info_t pwm_info;   mp_get_buffer_raise(pwm_buf_obj,   &pwm_info,   MP_BUFFER_WRITE);

    uint16_t *audio_in_ptr = (uint16_t *)audio_info.buf;
    uint32_t *pwm_out_ptr = (uint32_t *)pwm_info.buf; 

    const int num_audio_samples = audio_info.len / sizeof(uint16_t);
    const int num_pwm_words = pwm_info.len / sizeof(uint32_t); 
    
    // --- Constants ---
    const int PWM_TOP = 49;    // Must match Python
    const int PWM_CENTER = 25; // (PWM_TOP + 1) / 2
    const float MODULATION_DEPTH = 0.95f;

    // These must match your Python script's constants
    const uint32_t PWM_UPDATE_RATE = 5000000;
    // const uint32_t AUDIO_SAMPLE_RATE = 22050;
    const float RATIO = (float)PWM_UPDATE_RATE / (float)AUDIO_SAMPLE_RATE;

    int pwm_idx = 0;

    // --- Main Processing Loop ---
    for (int i = 0; i < num_audio_samples; i++) {
        // 1. Get current and next audio sample for linear interpolation
        float audio_start = ((float)audio_in_ptr[i] - 2048.0f) / 2048.0f;
        float audio_end = (i + 1 < num_audio_samples) ?
                          ((float)audio_in_ptr[i + 1] - 2048.0f) / 2048.0f :
                          audio_start;

        // 2. Linear Interpolation loop
        // Calculate how many PWM samples this one audio sample covers
        int start_j_idx = (int)(i * RATIO);
        int end_j_idx = (int)((i + 1) * RATIO);

        for (int pwm_sample_idx = start_j_idx; pwm_sample_idx < end_j_idx; pwm_sample_idx++) {
            if (pwm_idx >= num_pwm_words) break; // Safety break

            float interp_point = (float)(pwm_sample_idx - start_j_idx) / (float)(end_j_idx - start_j_idx);
            float audio_interp = audio_start * (1.0f - interp_point) + audio_end * interp_point;

            // 3. Generate carrier sample
            q31_t carrier_q31 = arm_cos_q31(self->tx_nco_phase);
            self->tx_nco_phase += self->tx_nco_phase_increment;
            float carrier_float = (float)carrier_q31 / 2147483648.0f;

            // 4. Modulate
            float modulator = 1.0f + (audio_interp * MODULATION_DEPTH);
            float am_signal = carrier_float * modulator;

            // 5. Scale to PWM duty cycle
            int32_t duty_cycle = (int32_t)(PWM_CENTER * (1.0f + am_signal));
            
            // 6. Clamp
            if (duty_cycle > PWM_TOP) duty_cycle = PWM_TOP;
            if (duty_cycle < 0) duty_cycle = 0;

            // 7. Pack two 16-bit duty cycles into one 32-bit word for the DMA
            pwm_out_ptr[pwm_idx] = ((uint32_t)duty_cycle << 16) | (uint32_t)duty_cycle;
            pwm_idx++;
        }
    }
    return mp_const_none;
}
// CORRECTED: Macro for a function with 3 args (self, audio_buf, pwm_buf)
static MP_DEFINE_CONST_FUN_OBJ_3(sdr_radio_am_transmit_pipeline_obj, sdr_radio_am_transmit_pipeline);



static const mp_rom_map_elem_t sdr_radio_locals_dict_table[] = {
    { MP_ROM_QSTR(MP_QSTR_tune), MP_ROM_PTR(&sdr_radio_tune_obj) },
    { MP_ROM_QSTR(MP_QSTR_reset_state), MP_ROM_PTR(&sdr_radio_reset_state_obj) },
 { MP_ROM_QSTR(MP_QSTR_set_mode), MP_ROM_PTR(&sdr_radio_set_mode_obj) },
 { MP_ROM_QSTR(MP_QSTR_fast_sdr_pipeline), MP_ROM_PTR(&fast_sdr_pipeline_obj) },
 { MP_ROM_QSTR(MP_QSTR_set_bfo), MP_ROM_PTR(&sdr_radio_set_bfo_obj) },

 { MP_ROM_QSTR(MP_QSTR_capture_chunk), MP_ROM_PTR(&sdr_radio_capture_chunk_obj) },
 { MP_ROM_QSTR(MP_QSTR_start_capture), MP_ROM_PTR(&sdr_radio_start_capture_obj) },
 { MP_ROM_QSTR(MP_QSTR_configure_and_init_capture), MP_ROM_PTR(&sdr_radio_configure_and_init_capture_obj) },

 { MP_ROM_QSTR(MP_QSTR_set_tx_carrier), MP_ROM_PTR(&sdr_radio_set_tx_carrier_obj) },
 { MP_ROM_QSTR(MP_QSTR_am_transmit_pipeline), MP_ROM_PTR(&sdr_radio_am_transmit_pipeline_obj) },
};


static MP_DEFINE_CONST_DICT(sdr_radio_locals_dict, sdr_radio_locals_dict_table);

const mp_obj_type_t sdr_radio_SDR_Radio_type;

MP_DEFINE_CONST_OBJ_TYPE( sdr_radio_SDR_Radio_type, MP_QSTR_SDR_Radio, MP_TYPE_FLAG_NONE, make_new, sdr_radio_make_new, locals_dict, &sdr_radio_locals_dict );

static const mp_rom_map_elem_t sdr_radio_module_globals_table[] = {
    { MP_ROM_QSTR(MP_QSTR___name__), MP_ROM_QSTR(MP_QSTR_sdr_radio) },
    { MP_ROM_QSTR(MP_QSTR_SDR_Radio), MP_ROM_PTR(&sdr_radio_SDR_Radio_type) },
    { MP_ROM_QSTR(MP_QSTR_deinit_capture), MP_ROM_PTR(&sdr_radio_deinit_capture_obj) },
 { MP_ROM_QSTR(MP_QSTR_configure_transmitter_pwm), MP_ROM_PTR(&configure_transmitter_pwm_obj) },
    { MP_ROM_QSTR(MP_QSTR_audio_configure), MP_ROM_PTR(&audio_configure_obj) },
    { MP_ROM_QSTR(MP_QSTR_audio_play_chunk), MP_ROM_PTR(&audio_play_chunk_obj) },
    { MP_ROM_QSTR(MP_QSTR_audio_wait_done), MP_ROM_PTR(&audio_wait_done_obj) },
    { MP_ROM_QSTR(MP_QSTR_audio_deinit), MP_ROM_PTR(&audio_deinit_obj) },
};

static MP_DEFINE_CONST_DICT(sdr_radio_module_globals, sdr_radio_module_globals_table);

const mp_obj_module_t sdr_radio_user_cmodule = { .base = { &mp_type_module }, .globals = (mp_obj_dict_t *)&sdr_radio_module_globals, };

#if MICROPY_PY_SDR_RADIO
MP_REGISTER_MODULE(MP_QSTR_sdr_radio, sdr_radio_user_cmodule);
#endif

That’s a lot of code to go over. But much of it is boilerplate used to connect C to MicroPython.

As a guide to explaining the C code, we will walk through the Python code for the radio. As you would expect, it is much smaller and simpler:

from array import array
from machine import Pin, ADC, PWM, freq
from sdr_radio import SDR_Radio
from sdr_radio import deinit_capture
from sdr_radio import audio_configure, audio_play_chunk, audio_wait_done, audio_deinit
from sys import print_exception

freq(250_000_000)

ADC_PIN = 26
PWM_PIN = 20
BUFFER_SIZE_SAMPLES = 8192
ADC_SAMPLE_RATE = 500000
AUDIO_SAMPLE_RATE = 22050
TARGET_FREQ = 810_000.0
BUFFERS_PER_SECOND = ADC_SAMPLE_RATE / BUFFER_SIZE_SAMPLES

led = Pin("LED", Pin.OUT)
led.off()

adc = ADC(Pin(ADC_PIN))
pwm = PWM(Pin(PWM_PIN))
audio_configure(pwm, AUDIO_SAMPLE_RATE)

DECIMATION_FACTOR = int(ADC_SAMPLE_RATE // AUDIO_SAMPLE_RATE)
audio_rate_len = BUFFER_SIZE_SAMPLES // DECIMATION_FACTOR

sdr = SDR_Radio()
sdr.configure_and_init_capture(ADC_SAMPLE_RATE, BUFFER_SIZE_SAMPLES)
sdr.start_capture()

def radio(f):
    sdr.set_mode(True)
    sdr.tune(int(f))

    adc_buf = array('H', (0 for _ in range(BUFFER_SIZE_SAMPLES)))
    pwm_long_bufs = [
        array('L', (0 for _ in range(audio_rate_len))),
        array('L', (0 for _ in range(audio_rate_len)))
    ]

    scratch_buf = array('l', (0 for _ in range(BUFFER_SIZE_SAMPLES * 2)))

    buf_idx = 0 
    sdr.capture_chunk(adc_buf)
    sdr.fast_sdr_pipeline([adc_buf, pwm_long_bufs[0], scratch_buf])
    audio_play_chunk(pwm, pwm_long_bufs[0])

    while True:
        next_buf_idx = 1 - buf_idx
        sdr.capture_chunk(adc_buf)
        sdr.fast_sdr_pipeline([adc_buf, pwm_long_bufs[buf_idx], scratch_buf])
        audio_play_chunk(pwm, pwm_long_bufs[buf_idx])
        buf_idx = next_buf_idx

def main():
    try:
        radio(810_000)
    except KeyboardInterrupt:
        print("\nUser interrupt.")
    except Exception as e:
        print("An exception occurred:")
        print_exception(e)
    finally:
        print("Cleaning up...")
        # Wait for any final chunk to finish before stopping
        audio_wait_done(pwm)
        audio_deinit(pwm)
        # This will stop the ADC and release DMA channels
        deinit_capture()
        led.value(0)
        print("Done.")

main()

Unlike the RTL-SDR, which has megahertz of bandwidth, our ADC can only run as fast as 500,000 samples per second.

But AM radio starts above that, at 530 kHz, and goes up from there to 1,700 kHz. How can we sample way up there?

We embrace aliasing. We sub-sample.

There’s a strong AM radio station in my area at 810 kHz. Sampling at 500 kHz gives me an alias frequency using this formula:

alias = 500 - (810 % 500)

The 500 is our sampling rate, and the 810 is the frequency we want to listen to. The result is 190 kHz. We need to tune to 190 kHz.

We create a sine wave (and a cosine wave) at 190 kHz and multiply our incoming ADC buffer by that. This “mixes” our target signal down to zero (DC). Since an AM signal carries the sound in two side-bands, we have just mixed the carrier down to DC and one sideband off the chart altogether, leaving the high sideband down in the audio range.

We get rid of the energy in the carrier (which is now DC) using a DC blocker (a high-pass filter). Then we get rid of the high-frequency aliases (that are above our hearing anyway) using a low-pass filter.

Now we have an audio signal, but it is sampled at 500 kHz, which is much too fast for our PWM, which would like 22050 Hz. So we “decimate”. We just keep every 22nd sample, and we’re at 22050.

To demodulate the AM signal, we would like to find the square root of the sum of the in-phase signal and the quadrature signal. These are the signals we created by mixing (respectively) the sine and the cosine waves.

But the square root subroutine takes too long. To our rescue comes the arm_math.h library, which is a set of highly optimised math routines that use special Arm instructions. The __QADD() routine does an approximation of the math we need.

Lastly, we filter once again to remove any thumping artifacts our math has created below the audio range. This is another high-pass filter.

Now we are ready for the Automatic Gain Control (AGC). We want weak faraway stations to sound as loud as strong nearby stations. If the signal is too strong, we reduce it. Too weak, and we increase it.

Now we want to send it to the PWM. The PWM wants to see 32-bit integers, where the high 16 bits are for the B channel, and the low 16 bits are for the A channel. Only the low byte is actually used for either channel. We send 128 to B, and our sample to A. This keeps the B channel quiet.

So, what does Python see?

After setting up the sizes and sample rates, it calls audio_configure() to set up the PWM DMA.

Then it gets an instance of the SDR_Radio object. This has the configure_and_init_capture() method to set up the ADC DMA.

Then it calls sdr.start_capture() to start filling the two “ping-pong” buffers for the DMA output. The ADC DMA is now running in the background, without bothering the CPU, filling first one buffer and then the other. This allows the CPU to read from the full buffer while the other one is being filled. Thus “ping pong”.

Next, we create a buffer for the ADC and two buffers for the PWM (so we can do a ping-pong with those in Python).

We “prime the pump” by capturing an ADC buffer, running it through our SDR pipeline to filter, mix, demodulate, filter, and AGC, and then finally play the audio using audio_play_chunk().

Once primed, we enter the main loop. We capture, pipeline, wait for the PWM to finish playing the primed audio, and then hand it another buffer to play.

While the buffer is playing, the ADC is collecting the next buffer of samples. The decimation ensures that the time it takes to play the audio is exactly the same time needed to fetch the next buffer of ADC samples.

While both of these are happening in the background, we have 16 milliseconds to process the buffer in the pipeline. Thanks to the fancy Arm math library, this only takes about 5 milliseconds, so we never stall the ADC or the PWM.

The code I presented above also includes the ability to handle CW and single-sideband reception, and it has an AM transmitter. But as this has been a long slog for you to read, we’ll play with those another day.

Our little RP2350, with only the help of a long wire and a speaker, is now a full-blown AM/CW/SSB radio.

And it only took six weeks of scull-sweat and about 5,000 re-compiles of the MicroPython code.