# Python Radio 16: MFSK Mode

Simon Quellen Field

Follow

20 min read

·

Sep 3, 2024

Listen

Share

More

Bigger and faster…

Press enter or click to view image in full size

Photo by the author

In the last article, we hinted at the Si5351 module that will allow us to send on any frequency we like, with a resolution of less than one Hertz.

In this article, we get to use it.

We will connect the Si5351 module (available for about a dollar on AliExpress, or $8 on Amazon.com) to the RP2040 using four wires.

The module uses the I2C interface, so all it needs are five-volt power, ground, a clock signal, and a data signal. The ESP32 and ESP8266 can also use this module, as they both support the I2C interface.

Like those microprocessors, the SI5351 also produces square waves by dividing a clock by integers, but it has much better resolution. Below 18 megahertz, it will never be more than half a hertz off of the target frequency. Even at 150 MHz the error is only 4 hertz, and 18% of the frequencies in that range don’t miss at all. The module has a nominal range of 8 kilohertz to 160 megahertz, but it can actually reach the 1.25-meter band up around 220 megahertz.

With the SI5351, we can transmit in modes that send multiple tones only a few hertz apart, such as MFSK.

As an added bonus, the SI5351 puts out 50 milliwatts of power, instead of the 3.8 milliwatts we got out of our raw RP2040. You can claim the 1,000 miles per watt award by having a conversation with someone 50 miles away, with no amplifier, powered by USB. QRP is anything 5 watts or less. QRPp is less than a watt, down to 100 milliwatts. Below that is called QRPpp, and our 50-milliwatt transmitter is in that class. You can, of course, add an amplifier if you feel the need for more power.

To use the module, we need a driver. Device drivers are difficult to understand without reading the entire specification sheet for the device, and so I won’t be going into any detail here. The following code is in the module SI5351.py:

from machine import I2C
import math

SI5351_REGISTER_0_DEVICE_STATUS                       = 0
SI5351_REGISTER_1_INTERRUPT_STATUS_STICKY             = 1
SI5351_REGISTER_2_INTERRUPT_STATUS_MASK               = 2
SI5351_REGISTER_3_OUTPUT_ENABLE_CONTROL               = 3
SI5351_REGISTER_9_OEB_PIN_ENABLE_CONTROL              = 9
SI5351_REGISTER_15_PLL_INPUT_SOURCE                   = 15
SI5351_REGISTER_16_CLK0_CONTROL                       = 16
SI5351_REGISTER_17_CLK1_CONTROL                       = 17
SI5351_REGISTER_18_CLK2_CONTROL                       = 18
SI5351_REGISTER_19_CLK3_CONTROL                       = 19
SI5351_REGISTER_20_CLK4_CONTROL                       = 20
SI5351_REGISTER_21_CLK5_CONTROL                       = 21
SI5351_REGISTER_22_CLK6_CONTROL                       = 22
SI5351_REGISTER_23_CLK7_CONTROL                       = 23
SI5351_REGISTER_24_CLK3_0_DISABLE_STATE               = 24
SI5351_REGISTER_25_CLK7_4_DISABLE_STATE               = 25
SI5351_REGISTER_42_MULTISYNTH0_PARAMETERS_1           = 42
SI5351_REGISTER_43_MULTISYNTH0_PARAMETERS_2           = 43
SI5351_REGISTER_44_MULTISYNTH0_PARAMETERS_3           = 44
SI5351_REGISTER_45_MULTISYNTH0_PARAMETERS_4           = 45
SI5351_REGISTER_46_MULTISYNTH0_PARAMETERS_5           = 46
SI5351_REGISTER_47_MULTISYNTH0_PARAMETERS_6           = 47
SI5351_REGISTER_48_MULTISYNTH0_PARAMETERS_7           = 48
SI5351_REGISTER_49_MULTISYNTH0_PARAMETERS_8           = 49
SI5351_REGISTER_50_MULTISYNTH1_PARAMETERS_1           = 50
SI5351_REGISTER_51_MULTISYNTH1_PARAMETERS_2           = 51
SI5351_REGISTER_52_MULTISYNTH1_PARAMETERS_3           = 52
SI5351_REGISTER_53_MULTISYNTH1_PARAMETERS_4           = 53
SI5351_REGISTER_54_MULTISYNTH1_PARAMETERS_5           = 54
SI5351_REGISTER_55_MULTISYNTH1_PARAMETERS_6           = 55
SI5351_REGISTER_56_MULTISYNTH1_PARAMETERS_7           = 56
SI5351_REGISTER_57_MULTISYNTH1_PARAMETERS_8           = 57
SI5351_REGISTER_58_MULTISYNTH2_PARAMETERS_1           = 58
SI5351_REGISTER_59_MULTISYNTH2_PARAMETERS_2           = 59
SI5351_REGISTER_60_MULTISYNTH2_PARAMETERS_3           = 60
SI5351_REGISTER_61_MULTISYNTH2_PARAMETERS_4           = 61
SI5351_REGISTER_62_MULTISYNTH2_PARAMETERS_5           = 62
SI5351_REGISTER_63_MULTISYNTH2_PARAMETERS_6           = 63
SI5351_REGISTER_64_MULTISYNTH2_PARAMETERS_7           = 64
SI5351_REGISTER_65_MULTISYNTH2_PARAMETERS_8           = 65
SI5351_REGISTER_66_MULTISYNTH3_PARAMETERS_1           = 66
SI5351_REGISTER_67_MULTISYNTH3_PARAMETERS_2           = 67
SI5351_REGISTER_68_MULTISYNTH3_PARAMETERS_3           = 68
SI5351_REGISTER_69_MULTISYNTH3_PARAMETERS_4           = 69
SI5351_REGISTER_70_MULTISYNTH3_PARAMETERS_5           = 70
SI5351_REGISTER_71_MULTISYNTH3_PARAMETERS_6           = 71
SI5351_REGISTER_72_MULTISYNTH3_PARAMETERS_7           = 72
SI5351_REGISTER_73_MULTISYNTH3_PARAMETERS_8           = 73
SI5351_REGISTER_74_MULTISYNTH4_PARAMETERS_1           = 74
SI5351_REGISTER_75_MULTISYNTH4_PARAMETERS_2           = 75
SI5351_REGISTER_76_MULTISYNTH4_PARAMETERS_3           = 76
SI5351_REGISTER_77_MULTISYNTH4_PARAMETERS_4           = 77
SI5351_REGISTER_78_MULTISYNTH4_PARAMETERS_5           = 78
SI5351_REGISTER_79_MULTISYNTH4_PARAMETERS_6           = 79
SI5351_REGISTER_80_MULTISYNTH4_PARAMETERS_7           = 80
SI5351_REGISTER_81_MULTISYNTH4_PARAMETERS_8           = 81
SI5351_REGISTER_82_MULTISYNTH5_PARAMETERS_1           = 82
SI5351_REGISTER_83_MULTISYNTH5_PARAMETERS_2           = 83
SI5351_REGISTER_84_MULTISYNTH5_PARAMETERS_3           = 84
SI5351_REGISTER_85_MULTISYNTH5_PARAMETERS_4           = 85
SI5351_REGISTER_86_MULTISYNTH5_PARAMETERS_5           = 86
SI5351_REGISTER_87_MULTISYNTH5_PARAMETERS_6           = 87
SI5351_REGISTER_88_MULTISYNTH5_PARAMETERS_7           = 88
SI5351_REGISTER_89_MULTISYNTH5_PARAMETERS_8           = 89
SI5351_REGISTER_90_MULTISYNTH6_PARAMETERS             = 90
SI5351_REGISTER_91_MULTISYNTH7_PARAMETERS             = 91
SI5351_REGISTER_092_CLOCK_6_7_OUTPUT_DIVIDER          = 92
SI5351_REGISTER_165_CLK0_INITIAL_PHASE_OFFSET         = 165
SI5351_REGISTER_166_CLK1_INITIAL_PHASE_OFFSET         = 166
SI5351_REGISTER_167_CLK2_INITIAL_PHASE_OFFSET         = 167
SI5351_REGISTER_168_CLK3_INITIAL_PHASE_OFFSET         = 168
SI5351_REGISTER_169_CLK4_INITIAL_PHASE_OFFSET         = 169
SI5351_REGISTER_170_CLK5_INITIAL_PHASE_OFFSET         = 170
SI5351_REGISTER_177_PLL_RESET                         = 177
SI5351_REGISTER_183_CRYSTAL_INTERNAL_LOAD_CAPACITANCE   = 183

SI5351_CRYSTAL_FREQ_25MHZ = 25000000
SI5351_CRYSTAL_FREQ_27MHZ = 27000000
SI5351_CRYSTAL_LOAD_6PF  = 1<<6
SI5351_CRYSTAL_LOAD_8PF  = 2<<6
SI5351_CRYSTAL_LOAD_10PF = 3<<6

si5351_15to92 = bytearray(b’\x00OOo\x80\x80\x80\x80\x80\x00\x00\x00\x05\x00\x0cf\x00\x00\x02\x02q\x00\x0c\x1a\x00\x00\x86\x00\x01\x00\x01\x00\x00\x00\x00\x00\x01\x00\x1c\x00\x00\x00\x00\x00\x01\x00\x18\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00\x00’)

SI5351_MULTISYNTH_DIV_4  = 4
SI5351_MULTISYNTH_DIV_6  = 6
SI5351_MULTISYNTH_DIV_8  = 8

class SI5351:
    def __init__(self, i2c, address=0x60, crystalFreq=25000000):
        self.i2c = i2c
        self.address = address
        
        self.initialized     = False
        self.crystalFreq     = crystalFreq
        self.crystalLoad     = SI5351_CRYSTAL_LOAD_10PF
        self.crystalPPM      = 30
        self.plla_configured = False
        self.plla_freq       = 0
        self.pllb_configured = False
        self.pllb_freq       = 0
        return

    def write8(self, register, value):
        ret = True
        self.i2c.start()   # only available in SoftI2C
        buffera = bytearray(1)
        buffera[0] = value & 0xff
        try:
          self.i2c.writeto_mem(self.address, register, buffera)
        except Exception as e:
          print(”Exception”, e, “when writing to Si5351”)
          print(”Address: “, self.address)
          print(”Scan: “, self.i2c.scan())
          ret = False
        self.i2c.stop()   # only available in SoftI2C
        return ret

    def read8(self, register, value):
        ret = True
        self.i2c.start()   # only available in SoftI2C
        buffera = bytearray(1)
        try:
          self.i2c.readfrom_mem_into(self.address, register, buffera)
        except Exception as e:
          print(”Exception”, e, “when writing to Si5351”)
          print(”Address: “, self.address)
          print(”Scan: “, self.i2c.scan())
          ret = False
        self.i2c.stop()   # only available in SoftI2C
        return ret


    def begin(self):
        self.write8(SI5351_REGISTER_3_OUTPUT_ENABLE_CONTROL, 0xFF)
        # Power down all output drivers */
        self.write8(SI5351_REGISTER_16_CLK0_CONTROL, 0x80)
        self.write8(SI5351_REGISTER_17_CLK1_CONTROL, 0x80)
        self.write8(SI5351_REGISTER_18_CLK2_CONTROL, 0x80)
        self.write8(SI5351_REGISTER_19_CLK3_CONTROL, 0x80)
        self.write8(SI5351_REGISTER_20_CLK4_CONTROL, 0x80)
        self.write8(SI5351_REGISTER_21_CLK5_CONTROL, 0x80)
        self.write8(SI5351_REGISTER_22_CLK6_CONTROL, 0x80)
        self.write8(SI5351_REGISTER_23_CLK7_CONTROL, 0x80)

        #  Set the load capacitance for the XTAL */
        self.write8(SI5351_REGISTER_183_CRYSTAL_INTERNAL_LOAD_CAPACITANCE,
                       self.crystalLoad)

        # Set interrupt masks as required (see Register 2 description in AN619).
        # By default, ClockBuilder Desktop sets this register to 0x18.
        # Note that the least significant nibble must remain 0x8, but the most
        # significant nibble may be modified to suit your needs. 

        # Reset the PLL config fields just in case we call init again
        self.plla_configured = False
        self.plla_freq = 0
        self.pllb_configured = False
        self.pllb_freq = 0

        # All done!
        self.initialized = True

        return

    def setClockBuilderData(self ):
        i = 0

        # Make sure we’ve called init first

        assert self.initialized == True, “you have not initialized the object”

        # Disable all outputs setting CLKx_DIS high
        self.write8(SI5351_REGISTER_3_OUTPUT_ENABLE_CONTROL, 0xFF)

        # Writes configuration data to device using the register map contents
        # generated by ClockBuilder Desktop (registers 15-92 + 149-170)
        for i, x in enumerate(range(15,93)): 
            #print(x,  si5351_15to92[i] )
            self.write8(x, si5351_15to92[i] )

        for i in range(149, 171):
            self.write8(i, 0x00) 

        # Apply soft reset
        self.write8(SI5351_REGISTER_177_PLL_RESET, 0xAC)

        # Enabled desired outputs (see Register 3)
        self.write8(SI5351_REGISTER_3_OUTPUT_ENABLE_CONTROL, 0x00)
        return None

    def setupPLL(self, mult, num, denom, pllsource = ‘A’, reset=True):
        assert self.initialized == True, “you have not initialized the object”
        assert ((mult > 14) and (mult < 91) ), “invalid mult parameter”
        assert denom > 0, “denom must be > 0”
        assert num <= 0xfffff, “invalid parameter num”
        assert denom <= 0xfffff, “invalid parameter denom”
        if num ==0:
            P1 = 128*mult -512
            P2 = num
            P3 = denom
        else:
            P1 = 128*mult + math.floor( 128 * num/denom ) -512
            P2 = 128*num - denom * math.floor( 128 * num/denom)
            P3 = denom 

        if pllsource == ‘A’:
            baseaddr = 26
        else:
            baseaddr = 34

        P1 = int(P1)
        self.write8( baseaddr,   (P3 & 0x0000FF00) >> 8)
        self.write8( baseaddr+1, (P3 & 0x000000FF))
        self.write8( baseaddr+2, (P1 & 0x00030000) >> 16)
        self.write8( baseaddr+3, (P1 & 0x0000FF00) >> 8)
        self.write8( baseaddr+4, (P1 & 0x000000FF))
        self.write8( baseaddr+5, ((P3 & 0x000F0000) >> 12) | ((P2 & 0x000F0000) >> 16) )
        self.write8( baseaddr+6, (P2 & 0x0000FF00) >> 8)
        self.write8( baseaddr+7, (P2 & 0x000000FF))

        if reset:
          self.write8(SI5351_REGISTER_177_PLL_RESET, (1<<7) | (1<<5) )

        if pllsource ==’A’:
            fvco = self.crystalFreq*( mult + num/denom)
            self.plla_configured = True
            self.plla_freq = int(math.floor( fvco))
        else:
            fvco = self.crystalFreq*(mult + num/denom)
            self.pllb_configured = True
            self.pllb_freq = int(math.floor(fvco))
        return None

    def setupRdiv( self, output, div):
        assert output in [0,1,2], “output value invalid”
        assert div in [1,2,4,8,16,32,64,128], “div invalid”
        divdict = {1: 0, 2: 1, 4: 2, 8: 3, 16: 4, 32: 5, 64: 6, 128: 7}
        registers = [ 44, 52, 60]
        Rreg = registers[output]
        buf = bytearray( 1)

        self.read8(Rreg, buf)

        regval = buf[0] & 0x0F
        divider = divdict[div]
        divider &= 0x07
        divider <<= 4
        regval |= divider
        self.write8(Rreg, regval)

        return None

    def setupMultisynth( self, output, div, num, denom, pllsource, power=3):
        assert self.initialized  == True, “device not initialized”
        assert output in [0,1,2], “output out of range”
        assert div > 3, “div out of range”
        assert denom >0, “denom out of range”
        assert num <= 0xfffff, “num has a 20-bit limit”
        assert denom <= 0xfffff, “denom as a 20-bit limit”
        if pllsource==”A”:
            assert self.plla_configured == True, “plla has not been configured”
        else:
            assert self.pllb_configured == True, ‘pllb has not been configured’

         # Output Multisynth Divider Equations
         # where: a = div, b = num and c = denom
         #
         #  P1 register is an 18-bit value using following formula:
         #
         # P1[17:0] = 128 * a + floor(128*(b/c)) - 512
         #
         # P2 register is a 20-bit value using the following formula:
         #
         #  P2[19:0] = 128 * b - c * floor(128*(b/c))
         #
         # P3 register is a 20-bit value using the following formula:
         #
         # P3[19:0] = c

        if num==0:
            # integer mode
            P1 = int(128 * div - 512)
            P2 = num
            P3 = denom
        else:
            # Fractional mode */
            P1 = int( 128 * div + math.floor(128 * (num/denom)) - 512 )
            P2 = int( 128 * num - denom * math.floor(128 * (num/denom)))
            P3 = denom


        baseaddrs = [ 42, 50, 58]
        baseaddr = baseaddrs[output]

        self.write8( baseaddr,  (P3 & 0x0000FF00) >> 8)
        self.write8( baseaddr+1, (P3 & 0x000000FF))
        self.write8( baseaddr+2, (P1 & 0x00030000) >> 16) # ToDo: Add DIVBY4 (>150MHz) and R0 support (<500kHz) later */
        self.write8( baseaddr+3, (P1 & 0x0000FF00) >> 8)
        self.write8( baseaddr+4, (P1 & 0x000000FF))
        self.write8( baseaddr+5, ((P3 & 0x000F0000) >> 12) | ((P2 & 0x000F0000) >> 16) )
        self.write8( baseaddr+6, (P2 & 0x0000FF00) >> 8)
        self.write8( baseaddr+7, (P2 & 0x000000FF))

         # Configure the clk control and enable the output 
        clkControlReg = 0x0F                             # 8mA drive strength, MS0 as CLK0 source, Clock not inverted, powered up 
        if pllsource == ‘B’:
            clkControlReg |= (1 << 5) # /* Uses PLLB */
        if num == 0:
            clkControlReg |= (1 << 6) #  Integer mode */

        #
        # Set power level bits
        # 0: 2mA -8dB
        # 1: 4mA -3dB
        # 2: 6mA -1dB
        # 3: 8mA -0dB (default)
        #
        # clkControlReg |= power & 3

        if output == 0: 
            self.write8(SI5351_REGISTER_16_CLK0_CONTROL, clkControlReg)
        if output == 1:
            self.write8(SI5351_REGISTER_17_CLK1_CONTROL, clkControlReg)
        if output == 2:
            self.write8(SI5351_REGISTER_18_CLK2_CONTROL, clkControlReg)


    def enableOutputs( self, enabled=True):
        assert self.initialized == True, “Error Device not initialized”
        if enabled:
            ret = self.write8( SI5351_REGISTER_3_OUTPUT_ENABLE_CONTROL, 0x00)
        else:
            ret = self.write8( SI5351_REGISTER_3_OUTPUT_ENABLE_CONTROL, 0xff)

        return ret

Our radio.py module handles the details of using the SI5351 to output square waves at the frequencies we choose. The SI5351 has two clocks, called A and B in the spec sheet, but we will refer to them as 0 and 1. For the most part, we will only deal with clock 0. Likewise, it has three outputs, but we will only use the first one, labeled CLK0.

Here is the code:

import utime

class Radio:
  def __init__(self):
    from SI5351 import SI5351
    from machine import Pin, SoftI2C

    # self.i2c = SoftI2C(scl=Pin(22), sda=Pin(21), freq=400000)     # ESP32
    self.i2c = SoftI2C(scl=Pin(1), sda=Pin(0), freq=400000)     # RP2040

    # self.i2c = SoftI2C(scl=Pin(22), sda=Pin(21),   freq=800000)     # The ESP32 can do up to 5 MHz best case
    print( “I2C.scan():”, self.i2c.scan())
    for x in range(5):
      self.clockgen = SI5351(self.i2c, 0x60 + x)
      status = 0
      if self.clockgen.read8(0, status):
        break
    self.clockgen.begin()
    self.clockgen.setClockBuilderData()
    self.key_state = False
    self.actual_freq_a = 0
    self.actual_freq_b = 0
    self.nominal_freq_a = 0
    self.nominal_freq_b = 0
    # self.last_time = utime.ticks_ms()

    self.old_mult = 0
    self.old_num = 0
    self.old_denom = 0
    self.old_src = 0

  def gcd(self, x, y):
     while(y): 
         x, y = y, x % y 
     return x 

  def send(self):
    pass

  def info(self):
    print( “I2C.scan():”, self.i2c.scan())
    
  def on(self):
    if self.clockgen.enableOutputs(True):
      self.key_state = True

  def off(self):
    if self.clockgen.enableOutputs(False):
      self.key_state = False

  def key_down(self):
    if self.clockgen.enableOutputs(True):
      self.key_state = True

  def key_up(self):
    if self.clockgen.enableOutputs(False):
      self.key_state = False

  def get_freq(self, which):
    if which == 0:
      return self.nominal_freq_a
    else:
      return self.nominal_freq_b

  def set_freq(self, which, f):
    f = float(f)
    div = int(900000000.0 / f)              # Values under a megahertz need an extra divide step
    r = 1
    while div > 900:
      r *= 2
      div /= 2
   
    if div % 2:                             # Make sure it is an even number
      div -= 1
   
    pllFreq = div * r * f
   
    xtal_freq = 25000000                    # Our board uses a 25 MHz crystal
    fmult = pllFreq / xtal_freq             # The full multiplier
    mult = int(fmult)                       # The integer part of the multiplier
    frac = fmult - mult
    off = int(frac * xtal_freq)
    divisor = self.gcd(off, xtal_freq)
    num = int(off / divisor)
    denom = int(xtal_freq / divisor)
   
    if num > 0xFFFFF or denom > 0xFFFFF:
      denom = 0xFFFFF                       # Use the maximum value for the denominator
      num = int((pllFreq % xtal_freq) * denom / xtal_freq)


    # Below 18 MHz, we will never be more than half a Hertz off
    # Below 37.5 MHz, we will never be more than a Hertz off
    # Below 75 MHz, we will never be more than two Hertz off
    # Below 112.5 MHz, we will never be more than three Hertz off
    # Below 150 MHz, we will never be more than four Hertz off
    # Below 222 MHz, we will never be more than six Hertz off
    # A little over 18% of the frequencies were right on the money
    # This is better than the frequency stability of a temperature controlled crystal oscillator
    # so any failure of accuracy here will be swamped by the variability in the oscillator
    # Of course, if you have a nice OCXO crystal oscillator in an oven that has a parts-per-billion accuracy
    # then you might want to know that you will never be off by more than 4 Hz in the 200 Hz wide WSPR window
    # in the 2 meter band.

    # All that assumes that we have double precision arithmetic. Micropython on the ESP8266 only has single precision 32 bit floats.
    # So we can think we are off by as much as 5 Hz in the 40-meter band, when the Si5351 is actually much more accurate.
    # On the ESP32, I have built special micropython firmware that supports double precision artithmetic.

    # If r is 1, we will never be less than our target frequency

    if which == 0:
      self.actual_freq_a = (mult * xtal_freq + xtal_freq * num / denom) / div * r
      self.nominal_freq_a = f
      src = "A"
    else:
      self.actual_freq_b = (mult * xtal_freq + xtal_freq * num / denom) / div * r
      self.nominal_freq_b = f
      src = "B"

    # print("Mult is", mult, "Num is", num, "Denom is", denom, "Src is", src, "Div is", div)

    reset = False
    if mult != self.old_mult and num != self.old_num and denom != self.old_denom and src != self.old_src:
      reset = True

    self.clockgen.setupPLL(mult, num, denom, pllsource=src, reset=reset)
    self.old_mult = mult
    self.old_num = num
    self.old_denom = denom
    self.old_src = src

    self.clockgen.setupMultisynth(output=0, div=div, num=0, denom=1, pllsource=src)
    if r > 1:
      self.clockgen.setupRdiv(output=0, div=r)

Most of the code is in the set_freq() method, and half of that is a comment.

The __init__() method handles setting up the I2C interface. We use the software version of I2C, because it is the same on all the microprocessors, so we don’t have to special case any code (other than which pins to use).

To get the extra resolution that we bought the module for, the code sets up more than the one integer divider we had in the RP2040. There is a numerator integer, a denominator integer, and a multiplier. There is an extra division step if the frequency is under a megahertz. All of this is so we have enough bits of precision to hit any frequency we wish in a large range.

MFSK stands for Multiple Frequency Shift Keying. Instead of just two tones, as we had in RTTY, sets of either 16 tones or 32 tones are used. The spacing between the tones is very narrow (such as 3.9065 hertz apart), so the bandwidth is quite low. Baud rates range from just under 4 to 125.

Unlike RTTY MFSK has a large character set, made up of 255 characters. All of the ASCII characters are there, as well as many characters for international keyboards, symbols for currency, degrees, and many more.

The characters are encoded in what is called “varicode”, where the more frequently used characters can be sent using fewer bits. Letters like “e” and “t” use only four bits. Characters such as “%” and “&” use ten bits.

Here is the whole table, in a module called mfsk_varicode.py:

# -*- coding: latin-1 -*-
mfsk_varicode = [
 0b11101011100,  # 000 - <NUL>
 0b11101100000,  # 001 - <SOH>
 0b11101101000,  # 002 - <STX>
 0b11101101100,  # 003 - <ETX>
 0b11101110000,  # 004 - <EOT>
 0b11101110100,  # 005 - <ENQ>
 0b11101111000,  # 006 - <ACK>
 0b11101111100,  # 007 - <BEL>
 0b10101000,     # 008 - <BS>
 0b11110000000,  # 009 - <TAB>
 0b11110100000,  # 010 - <LF>
 0b11110101000,  # 011 - <VT>
 0b11110101100,  # 012 - <FF>
 0b10101100,     # 013 - <CR>
 0b11110110000,  # 014 - <SO>
 0b11110110100,  # 015 - <SI>
 0b11110111000,  # 016 - <DLE>
 0b11110111100,  # 017 - <DC1>
 0b11111000000,  # 018 - <DC2>
 0b11111010000,  # 019 - <DC3>
 0b11111010100,  # 020 - <DC4>
 0b11111011000,  # 021 - <NAK>
 0b11111011100,  # 022 - <SYN>
 0b11111100000,  # 023 - <ETB>
 0b11111101000,  # 024 - <CAN>
 0b11111101100,  # 025 - <EM>
 0b11111110000,  # 026 - <SUB>
 0b11111110100,  # 027 - <ESC>
 0b11111111000,  # 028 - <FS>
 0b11111111100,  # 029 - <GS>
 0b100000000000,  # 030 - <RS>
 0b101000000000,  # 031 - <US>
 0b100,        # 032 - <SPC>
 0b111000000,    # 033 - !
 0b111111100,    # 034 - '"'
 0b1011011000,    # 035 -   #
 0b1010101000,    # 036 - $
 0b1010100000,    # 037 - %
 0b1000000000,    # 038 - &
 0b110111100,    # 039 - '
 0b111110100,    # 040 - (
 0b111110000,    # 041 - )
 0b1010110100,    # 042 - *
 0b111100000,    # 043 - +
 0b10100000,     # 044 - ,
 0b111011000,    # 045 - -
 0b111010100,    # 046 - .
 0b111101000,    # 047 - /
 0b11100000,     # 048 - 0
 0b11110000,     # 049 - 1
 0b101000000,    # 050 - 2
 0b101010100,    # 051 - 3
 0b101110100,    # 052 - 4
 0b101100000,    # 053 - 5
 0b101101100,    # 054 - 6
 0b110100000,    # 055 - 7
 0b110000000,    # 056 - 8
 0b110101100,    # 057 - 9
 0b111101100,    # 058 - :
 0b111111000,    # 059 - ;
 0b1011000000,    # 060 - <
 0b111011100,    # 061 - =
 0b1010111100,    # 062 - >
 0b111010000,    # 063 - ?
 0b1010000000,    # 064 - @
 0b10111100,     # 065 - A
 0b100000000,    # 066 - B
 0b11010100,     # 067 - C
 0b11011100,     # 068 - D
 0b10111000,     # 069 - E
 0b11111000,     # 070 - F
 0b101010000,    # 071 - G
 0b101011000,    # 072 - H
 0b11000000,     # 073 - I
 0b110110100,    # 074 - J
 0b101111100,    # 075 - K
 0b11110100,     # 076 - L
 0b11101000,     # 077 - M
 0b11111100,     # 078 - N
 0b11010000,     # 079 - O
 0b11101100,     # 080 - P
 0b110110000,    # 081 - Q
 0b11011000,     # 082 - R
 0b10110100,     # 083 - S
 0b10110000,     # 084 - T
 0b101011100,    # 085 - U
 0b110101000,    # 086 - V
 0b101101000,    # 087 - W
 0b101110000,    # 088 - X
 0b101111000,    # 089 - Y
 0b110111000,    # 090 - Z
 0b1011101000,    # 091 - [
 0b1011010000,    # 092 - \
 0b1011101100,    # 093 - ]
 0b1011010100,    # 094 - ^
 0b1010110000,    # 095 - _
 0b1010101100,    # 096 - `
 0b10100,       # 097 - a
 0b1100000,     # 098 - b
 0b111000,       # 099 - c
 0b110100,       # 100 - d
 0b1000,        # 101 - e
 0b1010000,     # 102 - f
 0b1011000,     # 103 - g
 0b110000,       # 104 - h
 0b11000,       # 105 - i
 0b10000000,     # 106 - j
 0b1110000,     # 107 - k
 0b101100,       # 108 - l
 0b1000000,     # 109 - m
 0b11100,       # 110 - n
 0b10000,       # 111 - o
 0b1010100,     # 112 - p
 0b1111000,     # 113 - q
 0b100000,       # 114 - r
 0b101000,       # 115 - s
 0b1100,        # 116 - t
 0b111100,       # 117 - u
 0b1101100,     # 118 - v
 0b1101000,     # 119 - w
 0b1110100,     # 120 - x
 0b1011100,     # 121 - y
 0b1111100,     # 122 - z
 0b1011011100,    # 123 - {
 0b1010111000,    # 124 - |
 0b1011100000,    # 125 - }
 0b1011110000,    # 126 - ~
 0b101010000000,  # 127 - <DEL>
 0b101010100000,  # 128 - 
 0b101010101000,  # 129 - 
 0b101010101100,  # 130 - 
 0b101010110000,  # 131 - 
 0b101010110100,  # 132 - 
 0b101010111000,  # 133 - 
 0b101010111100,  # 134 - 
 0b101011000000,  # 135 - 
 0b101011010000,  # 136 - 
 0b101011010100,  # 137 - 
 0b101011011000,  # 138 - 
 0b101011011100,  # 139 - 
 0b101011100000,  # 140 - 
 0b101011101000,  # 141 - 
 0b101011101100,  # 142 - 
 0b101011110000,  # 143 - 
 0b101011110100,  # 144 - 
 0b101011111000,  # 145 - 
 0b101011111100,  # 146 - 
 0b101100000000,  # 147 - 
 0b101101000000,  # 148 - 
 0b101101010000,  # 149 - 
 0b101101010100,  # 150 - 
 0b101101011000,  # 151 - 
 0b101101011100,  # 152 - 
 0b101101100000,  # 153 - 
 0b101101101000,  # 154 - 
 0b101101101100,  # 155 - 
 0b101101110000,  # 156 - 
 0b101101110100,  # 157 - 
 0b101101111000,  # 158 - 
 0b101101111100,  # 159 - 
 0b1011110100,    # 160 - 
 0b1011111000,    # 161 - ¡
 0b1011111100,    # 162 - ¢
 0b1100000000,    # 163 - £
 0b1101000000,    # 164 - ¤
 0b1101010000,    # 165 - ¥
 0b1101010100,    # 166 - ¦
 0b1101011000,    # 167 - §
 0b1101011100,    # 168 - ¨
 0b1101100000,    # 169 - ©
 0b1101101000,    # 170 - ª
 0b1101101100,    # 171 - «
 0b1101110000,    # 172 - ¬
 0b1101110100,    # 173 - 
 0b1101111000,    # 174 - ®
 0b1101111100,    # 175 - ¯
 0b1110000000,    # 176 - °
 0b1110100000,    # 177 - ±
 0b1110101000,    # 178 - ²
 0b1110101100,    # 179 - ³
 0b1110110000,    # 180 - ´
 0b1110110100,    # 181 - µ
 0b1110111000,    # 182 - ¶
 0b1110111100,    # 183 - ·
 0b1111000000,    # 184 - ¸
 0b1111010000,    # 185 - ¹
 0b1111010100,    # 186 - º
 0b1111011000,    # 187 - »
 0b1111011100,    # 188 - ¼
 0b1111100000,    # 189 - ½
 0b1111101000,    # 190 - ¾
 0b1111101100,    # 191 - ¿
 0b1111110000,    # 192 - À
 0b1111110100,    # 193 - Á
 0b1111111000,    # 194 - Â
 0b1111111100,    # 195 - Ã
 0b10000000000,  # 196 - Ä
 0b10100000000,  # 197 - Å
 0b10101000000,  # 198 - Æ
 0b10101010000,  # 199 - Ç
 0b10101010100,  # 200 - È
 0b10101011000,  # 201 - É
 0b10101011100,  # 202 - Ê
 0b10101100000,  # 203 - Ë
 0b10101101000,  # 204 - Ì
 0b10101101100,  # 205 - Í
 0b10101110000,  # 206 - Î
 0b10101110100,  # 207 - Ï
 0b10101111000,  # 208 - Ð
 0b10101111100,  # 209 - Ñ
 0b10110000000,  # 210 - Ò
 0b10110100000,  # 211 - Ó
 0b10110101000,  # 212 - Ô
 0b10110101100,  # 213 - Õ
 0b10110110000,  # 214 - Ö
 0b10110110100,  # 215 - ×
 0b10110111000,  # 216 - Ø
 0b10110111100,  # 217 - Ù
 0b10111000000,  # 218 - Ú
 0b10111010000,  # 219 - Û
 0b10111010100,  # 220 - Ü
 0b10111011000,  # 221 - Ý
 0b10111011100,  # 222 - Þ
 0b10111100000,  # 223 - ß
 0b10111101000,  # 224 - à
 0b10111101100,  # 225 - á
 0b10111110000,  # 226 - â
 0b10111110100,  # 227 - ã
 0b10111111000,  # 228 - ä
 0b10111111100,  # 229 - å
 0b11000000000,  # 230 - æ
 0b11010000000,  # 231 - ç
 0b11010100000,  # 232 - è
 0b11010101000,  # 233 - é
 0b11010101100,  # 234 - ê
 0b11010110000,  # 235 - ë
 0b11010110100,  # 236 - ì
 0b11010111000,  # 237 - í
 0b11010111100,  # 238 - î
 0b11011000000,  # 239 - ï
 0b11011010000,  # 240 - ð
 0b11011010100,  # 241 - ñ
 0b11011011000,  # 242 - ò
 0b11011011100,  # 243 - ó
 0b11011100000,  # 244 - ô
 0b11011101000,  # 245 - õ
 0b11011101100,  # 246 - ö
 0b11011110000,  # 247 - ÷
 0b11011110100,  # 248 - ø
 0b11011111000,  # 249 - ù
 0b11011111100,  # 250 - ú
 0b11100000000,  # 251 - û
 0b11101000000,  # 252 - n
 0b11101010000,  # 253 - ý
 0b11101010100,  # 254 - þ
 0b11101011000    # 255 - ÿ
]

The mfsk_config.py module connects the Radio and MFSK classes and isolates the rest of the program from their details. Most of it is concerned with setting up the nine different modes, each with a different baud rate and number of tones and symbols. Otherwise, it looks much like the config module for RTTY:

from mfsk import MFSK
from time import sleep_ms, sleep

from radio import Radio

class MfskProcess:
  def __init__(self, pin, frequency, baud, message, call, location):
    from machine import Timer
    
    self.osc = Radio()    
    self.radio_timer = Timer()
    self.old_tone = -1
    self.baud = baud
    self.usb_offset = 1133
    self.frequency = frequency
    self.message = message

    self.r = MFSK(self.radio_timer, self.send_tone)
    self.r.stop()
    self.r.set_call(call)
    self.r.set_location(location)
    self.r.set_frequency(frequency)
    self.r.set_message(message)

    if self.baud == 4:                    # 3.90625 baud
      self.r.samplerate = 8000.0
      self.r.symlen = 2048.0
      self.r.symbits = 5
      self.r.depth = 5
      self.r.basetone = 256
      self.r.numtones = 32
      self.r.preamble = 107
    elif self.baud == 8:                  # 7.8125 baud
      self.r.samplerate = 8000.0
      self.r.symlen = 1024.0
      self.r.symbits = 5
      self.r.depth = 5
      self.r.basetone = 128
      self.r.numtones = 32
      self.r.preamble = 107
    elif self.baud == 11:                 # 10.7666015625 baud
      self.r.samplerate = 11025.0
      self.r.symlen = 1024.0
      self.r.symbits = 4
      self.r.depth = 10
      self.r.basetone = 93
      self.r.numtones = 16
      self.r.preamble = 107
    elif self.baud == 16:                 # 15.625 baud
      self.r.samplerate = 8000.0
      self.r.symlen = 512.0
      self.r.symbits = 4
      self.r.depth = 10
      self.r.basetone = 64
      self.r.numtones = 16
      self.r.preamble = 107
    elif self.baud == 22:                 # 21.533203125 baud
      self.r.samplerate = 11025.0
      self.r.symlen = 512.0
      self.r.symbits = 4
      self.r.depth = 10
      self.r.basetone = 46
      self.r.numtones = 16
      self.r.preamble = 107
    elif self.baud == 31:                 # 31.25 baud
      self.r.samplerate = 8000.0
      self.r.symlen = 256.0
      self.r.symbits = 3
      self.r.depth = 10
      self.r.basetone = 32
      self.r.numtones = 8
      self.r.preamble = 107
    elif self.baud == 32:                 # 31.25 baud
      self.r.samplerate = 8000.0
      self.r.symlen = 256.0
      self.r.symbits = 4
      self.r.depth = 10
      self.r.basetone = 32
      self.r.numtones = 16
      self.r.preamble = 107
    elif self.baud == 64:                 # 62.5 baud
      self.r.samplerate = 8000.0
      self.r.symlen = 128.0
      self.r.symbits = 4
      self.r.depth = 10
      self.r.basetone = 16
      self.r.numtones = 16
      self.r.preamble = 180
    elif self.baud == 128:                # 125 baud
      self.r.samplerate = 8000.0
      self.r.symlen = 64.0
      self.r.symbits = 4
      self.r.depth = 20
      self.r.basetone = 8
      self.r.numtones = 16
      self.r.preamble = 214

    self.r.tonespacing = self.r.samplerate / self.r.symlen

    print(”Frequency:”, self.frequency)
    print(”Message:”, self.message)
 
    print(”Symbits is”, self.r.symbits)
    print(”Depth is”, self.r.depth)
    print(”Bandwidth is”, (self.r.numtones - 1) * self.r.tonespacing)
    print(”Symbol length is”, self.r.symlen)
    print(”Baud is”, self.r.samplerate / self.r.symlen)
    print(”Tonespacing is”, str(self.r.tonespacing) + “:”)
    self.send_code()

  def get_radio(self):
    return self.osc

  def set_message(self, msg):
    self.message = msg
    
  def send_code(self):
    self.f = float(self.r.basetone + float(self.frequency) + self.usb_offset)
    self.osc.set_freq(0, self.f)

    start_of_transmission_length = int(8 * (1000 / (self.r.samplerate / self.r.symlen)))
    sleep_ms(start_of_transmission_length)
    self.r.send_code()

  def send_tone(self, tone):
    if tone != self.old_tone:
      f = float(float(self.frequency) + self.usb_offset + float(tone))
      self.osc.set_freq(0, f)
    self.old_tone = tone

The mfsk.py module handles the actual encoding of the data. This is quite involved, as there is forward error correction to allow it to handle noisy environments. The error correction code was developed by NASA for space probes. The bits are also interleaved (high bits are sent, then next high, etc.) to allow the error correction code to handle bursts of static that would otherwise damage several bits in a row. The bits are also Gray coded, so that adjacent symbols differ by only one bit, which helps to reduce errors in the decoding.

The code looks like this:

from mfsk_varicode import mfsk_varicode
from machine import Timer

class MFSK:

  NASA_K = 7
  POLY1 = 0x6D
  POLY2 = 0x4F

  def __init__(self, timr, send_tone):
    self.timer = timr
    self.send_tone = send_tone

    #
    # Default is MFSK4
    #
    self.symbits = 5
    self.symlen = 2048
    self.samplerate = 8000
    self.depth = 5
    self.basetone = 256
    self.numtones = 32
    self.preamble = 107
    self.timer_running = False

    self.frequency = 7104000.0
    self.call = ”
    self.location = ”
    self.message = “{} {}   “
    self.count_tabs = 0
    self.has_bits = False
    self.sym_queue = []

    # Initialization for the forward error correction
    self.encoder_output = [0] * (1 << self.NASA_K)
    self.mask = (1 << self.NASA_K) - 1
    self.encode_state = 0
    self.bit_count = 0
    self.bit_state = 0

  # Code for the forward error correction
  def init_encoder(self):
    self.interleave_table = [8] * (self.symbits * self.symbits * self.depth)
    for x in range(1 << self.NASA_K):
      self.encoder_output[x] = (self.parity(self.POLY1 & x) | (self.parity(self.POLY2 &x) << 1))
    self.flush_interleave_table()

  # Hamming weight (the number of bits that are ones)
  def hamming_weight(self, w):
    w = (w & 0x55555555) + ((w >>  1) & 0x55555555)
    w = (w & 0x33333333) + ((w >>  2) & 0x33333333)
    w = (w & 0x0F0F0F0F) + ((w >>  4) & 0x0F0F0F0F)
    w = (w & 0x00FF00FF) + ((w >>  8) & 0x00FF00FF)
    w = (w & 0x0000FFFF) + ((w >> 16) & 0x0000FFFF)
    return w

  def parity(self, w):
    return self.hamming_weight(w) & 1

  def encode(self, bit):
    self.encode_state <<= 1
    if bit == “1”:
      self.encode_state |= 1

    return self.encoder_output[self.encode_state & self.mask]

  def set_call(self, call):
    self.call = call

  def set_baud(self, baud):
    self.baud = float(baud)

  def set_bit_length(self, len):
    self.bit_length = 1000000.0 / float(self.baud)

  def set_frequency(self, frequency):
    self.frequency = float(frequency)

  def set_location(self, location):
    self.location = location

  def set_message(self, message):
    self.message = message.format(self.call, self.location)
    self.message = “\r” + chr(2) + “\r” + self.message + “\r” + chr(0) + “\r”
    self.has_bits = True

  def bit(self):
    global mfsk_varicode

    for letter in self.message:
      code = mfsk_varicode[ord(letter) & 255]
      for bit in bin(code)[2:]:
        yield bit

  def stop(self):
    self.timer.deinit()
    self.timer_running = False
    self.all_done = True

  def send_code(self):
    self.set_baud(self.samplerate / self.symlen)
    self.bit_length = 1000000.0 / float(self.baud)
    self.tonespacing = self.samplerate / self.symlen
    self.bandwidth = (self.numtones - 1) * self.tonespacing
    self.init_encoder()

    self.all_done = False
    self.clearbits()
    self.gen = self.bit()
    if self.timer_running == False:
      self.timer.init(period=int(self.bit_length/1000), mode=Timer.PERIODIC, callback=self.next_tone)
      self.timer_running = True
    self.reported_end = False
    self.has_bits = True
    while self.has_bits:
      bit = self.get_bit()
      self.send_bit(bit)

    self.flush_tx(self.preamble)
    self.reported_end = True

  def get_bit(self):
    try:
      bit = next(self.gen)
    except StopIteration as e:
      self.has_bits = False
      return None
    return bit

  def send_bit(self, bit):
    try:
      data = self.encode(bit)
      for x in range(2):
        self.bit_state = (self.bit_state << 1) | ((data >> x) & 1)
        self.bit_count += 1

        if self.bit_count == self.symbits:
          self.interleave()
          self.send_symbol()
          self.bit_count = 0
          self.bit_state = 0
    except Exception as e:
      print(”Error:”, e)

  def clearbits(self):
    data = self.encode(0)
    for x in range(self.preamble):
      for y in range(2):
        self.bit_state = (self.bit_state << 1) | ((data >> x) & 1)
        self.bit_count += 1
        if self.bit_count == self.symbits:
          self.interleave()
          self.bit_count = 0
          self.bit_state = 0

  def interleave_get(self, x, y, z):
    index = self.symbits * self.symbits * x + self.symbits * y + z
    return self.interleave_table[index]

  def interleave_put(self, x, y, z, val):
    index = self.symbits * self.symbits * x + self.symbits * y + z
    self.interleave_table[index] = val

  def symbols(self):
    for x in range(self.depth):
      for y in range(self.symbits):
        for z in range(self.symbits - 1):
          self.interleave_put(x, y, z, self.interleave_get(x, y, z + 1))

      for y in range(self.symbits):
        self.interleave_put(x, y, self.symbits-1, self.syms[y])

      for y in range(self.symbits):
        self.syms[y] = self.interleave_get(x, y, self.symbits - y - 1)

  def interleave(self):
    self.syms = []
    for x in range(self.symbits):
      self.syms.append(self.bit_state >> ((self.symbits - x - 1)) & 1)

    self.symbols()

    self.bit_state = 0
    for x in range(self.symbits):
      self.bit_state = (self.bit_state << 1) | self.syms[x]

  def flush_interleave_table(self):
    for x in range(len(self.interleave_table)):
      self.interleave_table[x] = 0

  def flush_tx(self, preamble):
    self.send_bit(chr(1));
    for x in range(preamble):
      self.send_bit(chr(0));
    self.bit_state = 0
    self.all_done = True

  #
  # In order to reduce the number of bit errors in a digital modem,
  # all symbols are automatically Gray encoded such that adjacent
  # symbols in a constellation differ by only one bit.
  #
  def gray_encode(self, data):
    bits = data;
    bits ^= data >> 1;
    bits ^= data >> 2;
    bits ^= data >> 3;
    bits ^= data >> 4;
    bits ^= data >> 5;
    bits ^= data >> 6;
    bits ^= data >> 7;
    return bits;

  def send_symbol(self):
    from uasyncio import sleep_ms
    import utime
    sym = self.bit_state & (self.numtones - 1)
    sym = self.gray_encode(sym)
    while len(self.sym_queue) > 10:
      # 256000 / 500 is 512 milliseconds (2 symbols at 3.90625 baud)
      sleep_ms(int(self.bit_length / 500))           # Needed so the web server gets some time
      utime.sleep_ms(int(self.bit_length / 500))     # Needed so ^C works
    self.sym_queue.append(sym)

  def sendchar(self, ch):
    code = mfsk_varicode[ord(ch) & 255]
    for bit in bin(code)[2:]:
      self.send_bit(bit);

  def sendidle(self):
    self.sendchar(chr(0));

  def next_tone(self, unused):
    if self.sym_queue:
      sym = self.sym_queue.pop(0)
      self.send_tone(self.basetone + sym * self.tonespacing)

Finally, we get to our tiny little main.py module:

from mfsk_config import MfskProcess
from machine import Pin
from time import sleep
  
def main():
  mp = MfskProcess(15, 7040000, 4, ”, “AB6NY”, “CM87xe”)
  mp.set_message(”{} Testing from {} using a Raspberry Pi Pico RP2040”)
  
  while True:
    mp.send_code()
    while mp.r.all_done == False:
      sleep(5)

main()

It sets up pin 15 as the output, 7040000 as the frequency, and adds the call sign and Maidenhead Locator code to the message. Then it loops, sending the message over and over again, so that we can tune it in on the receiver.

The free software Fldigi program can receive and decode MFSK signals from your RTL-SDR. This makes testing much easier.

Mfsk

Python Programming